Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AERODYNAMICS										       Time: 3 hours

Subject Code: 11AE205						       		    		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the significance of Source and Sink in a Uniform flow?

2.	What is significance of Momentum equation?

3.	What is the Kutta condition?

4.	What is D’Alembert’s Paradox?

5.	What is Complex Velocity?

6.	What is the Cauchy-Riemann relation?

7.	Define Reynold’s number.

8.	Define Displacement thickness.

9.	What is bound Vortex?

10.	What is Horseshoe Vortex?

PART – B  (5 x 3 = 15 MARKS)

11.	Define Rankine Body. Give the potential function associated with a Rankine body.

12.	What is the Kutta-Joukowskii Theorem?

13.	Define Joukowski transformation. How can it be used to generate a cambered airfoil?

14.	What are the basic assumptions involved in deriving Boundary Layer equations from Navier-Stokes equations?

15.	State Kelvin’s Theorem.

PART – C  (5 x 15 = 75 MARKS)

16.	For a velocity field given by u(x,y)=Bx2−y2(x2+y2)2, v(x,y)=2B x y(x2+y2)2, find out whether 	the continuity equation is satisfied for an incompressible flow or not. If the continuity equation 	is satisfied, find the pressure distribution so as to satisfy the momentum equation.

(OR)

17.	Derive the continuity equation in differential form using control volume approach in the cylindrical coordinate system in the (r,θ) plane.

18.	Estimate the lift of a rotating cylinder in a uniform cross flow stream. The density of flow is ρ, free stream velocity U∞, radius of cylinder R and angular speed Ω.

(OR)

19.	Consider a circular cylinder of radius 50 cm in a cross flow with free stream velocity 100 m/s and density 1.2 kg/m3. The cylinder consists of two semi-circular parts forming forward and rear part. Find out the velocity and pressure distribution over the surface of the cylinder. Find: a.	Overall Drag of the cylinder 	b.	the force needed to separate the two parts of the cylinder.

[P.T.O]

20.	a.	If the circle ς= r is transformed using the Joukowski transformation z=ς+l2ς, show that 	transformed contour is an ellipse. Find its semi-major and semi-minor axis.

b.	If the circle ς=a+b2  is transformed to an ellipse with semi-major and semi-minor axes as a and b, find the relation between a, b and l.

(OR)

21.	Using the Joukowski transformation, derive the velocity field over a symmetric airfoil and find its lift coefficient.

22.	For an incompressible boundary layer over a flat plate with zero pressure gradient, derive Blausius equation for similar solution of a boundary layer.

(OR)

23.	For a laminar boundary layer velocity profile given by uUe=2η−2η2+η3,where η=yδ and δis boundary layer thickness, find out displacement and momentum thickness.

24.	Starting from the relation between circulation and angle of attack for an airfoil, derive the governing equation for distribution of circulation along the span of a finite wing (lifting line theory equation).

(OR)

25.	For a horseshoe vortex  in x-y plane as shown below, derive the induced velocity at an arbitrary point ( x,y,z).