Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SOLID STATE CIRCUITS							           	       Time: 3 hours

Subject Code: 11EC201				    				        	      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the function of a rectifier?

2.	What is the difference between load regulation and line regulation?

3.	Why do we choose Q point at the center of the load-line?

4.	What is meant by biasing?

5.	What is the efficiency of a Class-A power amplifier?

6.	What is the bandwidth of a video amplifier?

7.	What are the types of feed back?

8.	What is meant by noise?

9.	What are the conditions for a sustained oscillator?

10.	What is the classification of tuned amplifiers?

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the expression for ripple factor of diode half-wave rectifier.

12.	What are the requirements for biasing circuits?

13.	Define the terms collector efficiency, distortion and power dissipation capability as      	applied to Power Amplifiers.

14.	Explain how a negative feedback improves stability, reduces noise 	and increases input 	impedence.

15.	What are the classifications of Oscillators?

PART – C  (5 x 15 = 75 MARKS)

16.	With suitable sketches explain the operation of L section and π section filter.

(OR)

17.	Explain different protection circuits for voltage regulators.

18.	Discuss in detail the different Bipolar Junction Transistor biasing circuits.

(OR)

19.	a.	In a self biased CE amplifier circuit comprising 	of:											R2=90kΩ, R1=10kΩ, Vcc=22.5V, Rc=5.6kΩ and RE=1.0 kΩ ,

and a BJT with β0=55 and VBE=0 V find the operating  point and stability factor of the circuit. 																		(8)

b.	Determine the resistor RBfor a fixed bias and collector to base bias and compare the

two stability factors.   VCC=12V, RL=33 Ω, IB=0.3mA, β=100, VCE=6V.

(7)

[P.T.O]

20.	Explain the working of a class-B Push-pull Power amplifier with the help of a neat circuit diagram and relevant waveforms. What is its main disadvantage? Derive the expression for its efficiency.

(OR)

21.	Explain the operation of

a.	RC coupled  amplifier.  														(7)                                             	b.	Transformer coupled amplifier  												(8)

with neat circuit diagrams.

22.	Describe in detail about voltage shunt feedback amplifiers.

(OR)

23.	Discuss about current series feedback amplifiers in detail.

24.	Draw the circuit of an R-C Phase shift oscillator and explain its operation. Derive the 	expression for its frequency of oscillation and hence find the value of β and AV.

(OR)

25.	Draw the circuit of:

a.	Hartley oscillator.

b.	Colpitt’s Oscillator.

Explain their operations. Derive the expressions for their frequency of oscillation.