Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: NETWORK ANALYSIS AND SYNTHESIS				       	       Time: 3 hours

Subject Code: 11EC202			       					          	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define ‘voltage transfer ratio’ of a two-port network.

2.	The network function is said to be stable when real parts of the poles and zeros are 	___________.

3.	The steady state response of a network function is given by the equation _________.

4.	State the Routh criterion for stability.

5.	Define the term ‘co-tree’ used in graph theory.

6.	The cut-set schedule gives the relation between __________.

7.	Which parameters are widely used in transistor circuits?

8.	In a two-port bilateral network, if three transmission parameters are known, the fourth 	parameter can be determined from the equation __________.

9.	The roots of the odd and even parts of a Hurwitz polynomial lie on ___________.

10.	If there is pole at ________, the first element L0 is present in the second Foster form of an LC network.

PART – B (5 x 3 = 15 MARKS)

11.	Mention the significance of poles and zeros of the network function.

12.	State the importance of the magnitude and phase of the network functions in the network design.

13.	Explain the incidence matrix with a suitable example.

14.	Explain the attenuation characteristics of an ideal high pass filter.

15.	Check the positive realness of the function   Z(s)=s+3s+1  .

PART – C (5 x 15 = 75 MARKS)

16.	a. 	Explain the properties of transfer functions.                                                          		(8)

b. 	Obtain the driving point impedance function for the network shown in the figure below 	and find the 	poles and zeros.                                                           					(7)

(OR)

17.	The transform current I(s) in a network is given byI(s)=2s(s+1)(s+2). Draw the pole-

zero diagram and obtain the time domain response i(t).

[P.T.O]

18.	a.	For the given denominator polynomial of a network function, determine the value of K for 	which the network is stable. Q(s)=s3+2s2+4s+K.                                           		(8)

b. 	Sketch the complex locus plot of the admittance function Y(s), for the network shown in the figure below.                                                                                                     		(7)

(OR)

19.	Draw the frequency response plots of the voltage ratio transfer function of the two-port RC network shown in the figure below.

20. 	Draw the graph of the network shown in the figure below. Select a suitable tree to write the Tie-set schedule. Then find the three loop currents.

(OR)

21.	a. 	What are equilibrium equations? Explain the formulation of equilibrium equation on node  	basis with a suitable example.                                                                               		(8)

b. 	Draw the dual network for the network shown in the figure below.                              	(7)

22.	Find the Y-parameters for the network shown in the figure below.

[P.T.O]

(OR)

23. 	a. 	Obtain the lattice equivalent of a symmetrical T-network shown in the figure below.                 																					(8)

b. 	Design a high pass filter with a cut-off frequency of 1 KHz with a terminated design 	impedance of 800 ohm.                                                                                               	(7)

24.	a. 	Explain the properties of RL impedance and admittance functions.                            	(8)

b. 	Test whether the following polynomial is Hurwitz.

P(s)=s4+s3+5s2+3s+4.                                                                          			 (7)

(OR)

25. 	Find the first and second Foster forms of RC network for the function:

Z(s)=3(s+2)(s+4)(s+1)(s+3)