Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL ELECTRONICS								       	       Time: 3 hours

Subject Code: 11EC204			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Simplify A(A'+B).

2.	Convert the hexadecimal number 68BE to binary.

3.	How many output lines does an octal encoder have?

4.	What is the output of an even parity checker for input 1100?

5.	What is the function of a D flip-flop?

6.	What is a state diagram?

7.	What is a register?

8.	What is triggering source in ripple counter?

9.	Write different types if ROM.

10.	Write different output configurations of TTL gates.

PART – B (5 x 3 = 15 MARKS)

11.	Express the Boolean function F=AB+A'C in a product of maxterms form.

12.	Realize Half-adder with minimum number of logic gates.

13.	What is edge triggering? What are different types of edge triggering?

14.	What is the difference between serial and parallel transfer? What type of register is used in each case?

15.	What is a PLD?

PART – C (5 x 15 = 75 MARKS)

16.	Obtain the minimum sum of products for the function shown below using Quine McClausky 	method.

F=m0 + m2 + m4 + m8 + m9 + m10 + m11 + m12 + m13

(OR)

17.	a.	Prove the following Boolean identities.						       					(8)

i.	x+xyz+yz {x'+wx+w'x+x'y=x+y

ii.	(x1+x2)(x1′x3′+x3)(x2′+x1x3)'=x1′x2.

b. 	Find SOP of the function                                                                                            	(7)

F(x1,x2,x3,x4,x5)=∑m(1,3,6,10,11,12,14,15,17,19,20,22,24,29,30)

18.	Realize the Boolean function F=∑(0,1,2,4,5,7,8,9,12,13) using:

a.	8 to 1 multiplexer		b.	Decoder and external gates

(OR)

19.	a.	Implement a full adder circuit using:	i.	Decoder  	ii.	Multiplexer                 	(10)

b.	How can you convert a decoder into a demultiplexer?	                                          		(5)

[P.T.O]

20.	A sequential circuit has four flip flops A, B, C and D and an input x is described by the 	following state equations:                                                                                              				A(t+1)=(CD'+C'D)x+(CD+(CD {)')x; B(t+1)=A; C(t+1)=B; D(t+1)=C

- Obtain the sequence of states when x=1 starting from state ABCD=0001.

- Obtain the sequence of states when x=0 starting from state ABCD=0000.

(OR)

21.	A sequential circuit with 2 D flip flops A and B and input X and output Y is specified by the following next state and output equations.

A(t+1)=AX+BX; B(t+1)=A'X; Y=(A+B)X'

- Draw the logic diagram of the circuit.

- Derive the state table.

- Derive the state diagram.

22.	Design a synchronous decade counter using D flips flop.

(OR)

23.	Design an asynchronous decade counter using JK flip flop.

24.	Discuss on the concept of working and application of following memories:

a.	ROM		b.	EPROM			c.	PLA

(OR)

25.	Write notes on TTL, ECL and CMOS digital logic families.