Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: TRANSMISSION LINES AND WAVE GUIDES			       	       Time: 3 hours

Subject Code: 11EC212				       						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define skin depth.

2.	What is reflection coefficient?

3.	State the disadvantage of single stub matching.

4.	Give one possible arrangement to achieve impedance matching.

5.	E waves are otherwise called as _____________.

6.	Define attenuation factor.

7.	What is the cut-off frequency of the TE10 mode in a rectangular waveguide?

8.	Write down the expression for wave impedance of TM mode.

9.	Mention any one application of circular wave guides.

10.	A circular wave guide will behave like ___________filter.

PART – B (5 x 3 = 15 MARKS)

11.	Define wavelength. Give its expression.

12.	When are standing waves obtained? Give the expression for standing wave ratio.

13.	What is dominant mode? Give an example.

14.	What is a waveguide?

15.	Write the expression for Q factor of a waveguide.

PART – C (5 x 15 = 75 MARKS)

16.	With necessary diagrams derive the equations for a simple transmission line and a transmission line with load impedance.

(OR)

17.	Obtain the expression for an infinite transmission line.

18.	What is single-stub matching? Derive the expression for it.

(OR)

19.	a.	The characteristic impedance of a transmission line is 310 Ω and the transmission line is 	terminated by a pure resistive load. It is found by measurement that the minimum value of 	voltage upon the transmission line is 6 µV and the maximum voltage upon it is 9.5 µV. 	What is the value of load resistance?											(9)

b.	Write a note on impedance matching devices.									(3)

c.	What is a quarter-wave transformer?											(3)

20. 	Elaborate on the characteristics of TE and TM waves in parallel planes.

(OR)

21. 	Derive the expression for the attenuation factor of TE waves in parallel planes.

[P.T.O]

22. 	A rectangular wave guide measures 3 x 5 cm internally and has a 10 GHz signal propagated in it. Calculate:

a.	Cut-off wave length.

b.	Guide wave length.

c.	Characteristic wave impedance for the TE10 Mode.

(OR)

23. 	Write short notes on the following:

a.	Dominant mode in rectangular waveguides.										(8)

b.	Degenerated mode in rectangular waveguides.									(7)

24. 	With neat diagrams, describe the excitation of modes in circular waveguides.

(OR)

25. 	Given a circular waveguide used for a signal at a frequency of 11 GHz propagated in the TE01                       mode whose internal diameter is 4.5 cm, calculate:

a.	Cut-off frequency.					b.	Guide wavelength.

c.	Group velocity.						d.	Phase velocity.

e.	Characteristic impedance.