Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTROMAGNETIC FIELDS							       Time: 3 hours

Subject Code: 11EE206						       		    		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Express point P(–2,6,3) in cylindrical coordinates.

2.	Find the divergence of x2yz {ax+xy2zay+xyz2az.

3.	State Coulomb’s law for point charges.

4.	A Plane z=10 m carries charge 20 nC/m2. Find the electric field intensity at the origin.

5.	A static magnetic field is_________ (conservative/solenoidal).

6.	At the boundary between two magnetic media the normal component of the magnetic flux density is __________ (continuous/discontinuous).

7.	Eddy currents occur because the magnetic materials conduct electricity. (True/False).

8.	The flux through each turn of a 100-turn coil is (t3-2t) mWb, where t is in seconds. Find the induced emf at t=2 seconds.

9.	The electric field in free space is given by E=50cos(108t+3x)ay. Find the direction of wave propagation.

10.	In a lossless charge free medium a plane electromagnetic wave gets attenuated as it propagates. (True/False).

PART – B  (5 x 3 = 15 MARKS)

11.	If A=2ax+4ay and B=6ay−4az find the smaller angle between them using the cross product.

12.	Find the charge defined in a volume defined by 0≤x≤1m; 0≤y≤1m; and 0≤z≤1m; if  the volume charge density v=30x2y.

13.	Use Ampere’s law to obtain the magnetic field intensity H due to an infinitely long straight filament I.

14.	A circular cross section conductor of radius 1.5 mm carries a current ic=5.5sin(4x1010t)µA. 	What is the amplitude of the displacement current density if =35MS/m and r=1.

15.	Write down the 4 Maxwell’s equations for time varying fields in the phasor form.

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate the circulation of A=ρcosφaρ+zsinφaz around the edge defined by 0≤ρ≤2, 0≤φ≤600 , and z=0. Also verify Stokes’ Theorem.

(OR)

17.	Define the gradient of a scalar field. Prove that the maximum space rate of increase of a scalar function is in the direction of the gradient.

18.	Point charges 5nC and –2nc are located at (2, 0, 4) and (–3, 0, 5) respectively.

a.	Determine the force on a 1nC point charge located at (1,–3, 7).

b.	Find the electric field E at (1,–3, 7).

[P.T.O]

(OR)

19.	Derive an expression for the electric field at a point due to an infinite sheet of charge.

20.	Define Biot-Savart’s law. Derive an expression for the magnetic field at a point due to a straight filamentary conductor carrying current I.

(OR)

21.	A solenoid with turns N1=1000 and radius r1=1cm and length l1=50cm is concentric with a second coil of N2=2000, r2=2 cm and l2=50cm. Find the mutual inductance assuming free-space conditions.

22.	Define Faraday’s law of electromagnetic induction. Describe with suitable diagrams the concept of transformer emf and motional emf.

(OR)

23.	A parallel plate capacitor with plate area 5 cm2  and plate separation of 3mm has a voltage 50sin103t V applied to its plates. Calculate the displacement current assuming 0=2r.

24.	From the expressions for the attenuation constant  and wave number  for a lossy dielectric, obtain the expression for velocity of propagation, wavelength and intrinsic impedance for               a.	lossless dielectric 	b.	good conductor.

(OR)

25.	State and prove the Poynting Theorem.