Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DC MACHINES AND TRANSFORMERS				      	       Time: 3 hours

Subject Code: 11EE207			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A High voltage D.C. Machine uses ________ winding

2.	Draw the External Characteristic of a D.C. generator

3.	The back EMF in a D.C. Motor opposes the applied voltage. (True/False)

4.	In interpoles compensating windings neutralize the ________.

5.	Copper  loss varies ________ with power factor.

6.	Transformers are rated in kVA instead of kW because of ________.

7.	No-load test on a transformer is carried out to determine ________.

8.	When is the ordinary efficiency of a given transformer maximum?

9.	Which connection is the best suited for a 3 phase, 4 wire service?

10.	The secondary of a ________ transformer should never be left open.

PART – B  (5 x 3 = 15 MARKS)

11.	State the causes of heating of the armature in D.C.Machines.

12.	A 230 V DC shunt motor has an armature resistance of 0.5 ohm and field resistance of 115 ohm. At no load, the speed is 1200 rpm and the armature current 2.5 A. On application of rated load, the speed drops to 1120 rpm. Determine the line current and power input when the motor delivers rated load.

13.	Write notes on various losses in transformers.

14.	A 40 kVA, 1-ph transformer has an iron loss of 400 W, and full copper loss of 800 W. Find the load at which maximum efficiency is achieved at unity power factor.

15.	Draw the various types of three phase transformer connections.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the EMF equation of a D.C. Generator and obtain the condition for its maximum

efficiency.																	(8)

b.	A shunt generator supplies 96 A at a terminal voltage of 200 V. The armature and shunt

field resistances are 0.1 ohm and 50 ohm respectively. The iron and frictional losses are 2500W. 																		(7)

Find	i.	EMF generated		ii.	Copper loss		iii.	Commercial efficiency

(OR)

17.	Draw and brief about the constructional details of a D.C. generator and classify its types.

18.	a.	Draw and explain the electrical and mechanical characteristics of a DC series and

shunt motor.									               						(8)

b.	A 220 V DC Series motor runs at 900 rpm when taking a line current of 40A. The 				armature resistance and series field resistance are 0.06 ohm and 0.04 ohm respectively. If

current taken remains the same, calculate the series resistance required to reduce the speed to 600 rpm. 																	(7)

(OR)

[P.T.O]

19.	a.	Describe the working of a 3 point starter for DC shunt motor with a neat diagram. 	(8)

b.	A 2 pole series motor runs at 707 rpm when taking 100A at 85V and with the filed coils in series. The resistance of each filed coil is 0.03 ohm and that of the armature 0.04 ohm. If the filed coils are connected in parallel and load torque remains constant, find:

i.	speed		ii.	The additional resistance to be inserted in series with the motor to restore the speed to 707 rpm.													(7)

20.	a.	Derive the EMF equation and  condition for maximum efficiency in a transformer.	(8)

b.	A 11000/230 V, 150 kVA , 1-phase , 50 Hz transformer has core loss of 1.4 kW  and  F.L 		Cu loss of 1.6 kW. Determine:		i.	The kVA load for maximum efficiency and the 			value of 	maximum efficiency at unity p.f  	ii.	The efficiency at half F.L 0.8 pf leading 																					(7)

(OR)

21.	a.	Write short notes about the following: Voltage regulation, phasor diagram and equivalent 		circuit of the transformer  														(8)

b.	A 440/110 V transformer has a primary resistance of 0.03 ohm and secondary resistance of

0.02 ohm. Its iron loss at normal input is 150 W. Determine the secondary current at which

maximum efficiency will occur and the value of this maximum efficiency at a unity p.f load.																		(7)

22.	Brief about:																		(8+7)

a.	Sumpner’s test on  a transformer		b.	Operation of an auto transformer

(OR)

23.	Consider a 4 kVA, 200/400 V single phase transformer supplying full load current at 0.8 lagging power factor.  The O.C/S.C test results are as follows:

O.C.Test	 : 200V 		0.8 A	 	70 W	 	L.V.Side

S.C test		 : 20 V		10A			60W		H.V.Side

Calculate efficiency and voltage regulation for the above load.

24.	Write short notes on:															(7.5+7.5)

a.	Potential transformer			b.	Current transformer

(OR)

25.	a.	Illustrate the constructional details of a three phase transformer.					  (8)

b.	Brief about the parallel operation of a three phase transformer.						  (7)