Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CONTROL SYSTEMS									      	       Time: 3 hours

Subject Code: 11EE219				       		        		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the components of a feedback control system?

2.	Write the transfer function of a negative feedback system.

3.	Define Order of a system.

4.	Sketch the response of a second order under damped system.

5.	Define Phase Cross-Over Frequency.

6.	What is a non-minimum phase transfer function?

7.	What is the necessary condition for stability?

8.	What are asymptotes?

9.	What is a state?

10.	What are Phase variables?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the force balance equation of ideal mass element, ideal dashpot and ideal spring.

12.	A second order system has a damping ratio of 0.6 and natural frequency of oscillation is 10 rad/sec. Determine the damped frequency of oscillation, rise time and peak time of the system.

13.	Define Gain Margin and Phase Margin.

14.	State Nyquist Stability Criterion.

15.	State the properties of State Transition Matrix.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the transfer function of an armature controlled DC motor.

(OR)

17.	Obtain the transfer function for the following signal flow graph.

-H1

G4

G2							G8	     G6

G1

R(s)

Y(s)

G3								    G7

G5

-H2

[P.T.O]

18. 	A unity feedback system is characterized by the open-loop transfer functionG(s)=10s(0.1s+1). Evaluate the static error constants of the system. Obtain the steady state error of the system when subjected to an input given by the polynomial

r(t)=a0+a1t+a22t2.

(OR)

19.	A Unity feedback system is characterized by an open loop transfer function G(s)=ks(s+10).
Determine the gain K so that the system will have a damping ratio of 0.5 for this value of K.     Determine the settling time; peak overshoot and time to peak overshoot for a unit step input.

20.	Sketch the Bode plot for the following transfer function and determine the system gain margin and phase margin.

G(s)=10s(1+0.4s)(1+0.1s).

(OR)

21.	The open loop transfer function of a unity feedback system is given by G(s)=1s(1+s)(1+2s). Sketch the polar plot and determine the gain margin and phase margin.

22.	The characteristic polynomial of a system s7+5s6+9s5+9s4+4s3+20s2+36s+36=0. Determine the location of roots on the s-plane and hence the stability of the system.

(OR)

23.	A unity feedback control system has an open loop transfer function G(s)=Ks(s2+4s+13).

Sketch the root locus.

24.	Determine the state model of a field controlled DC motor.

(OR)

25.	A feedback system has a closed loop transfer functionY(s)U(s)=10(s+4)s(s+1)(s+3).  Construct two state models and give the block diagram representation for each state model.