Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL AND ELECTRONIC INSTRUMENTATION	      Time: 3 hours

Subject Code: 11EI201						       		        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The scale of a galvanometer is placed at a distance of 0.4 m from the mirror. A deflection of 44 mm is observed.  What is the angle through which the coil has turned?

2.	Why is current coil made fixed in the case of a dynamometer type wattmeter?

3.	Name the methods used for measurement of very high resistance.

4.	State the balance equation used in AC bridges.

5.	Why do electronic voltmeters which use rectifiers employ negative feedback?

6.	A wave analyzer acts as a frequency selective voltmeter. Justify the statement.

7.	In a CRO, the signals to be measured are applied to the ___________ plates whereas the sweep voltage is fed to the ___________ plates.

8.	Mention the use of Sampling Oscilloscopes.

9.	Digital instruments have input impedance of the order of ______________.

10.	What are the basic components of a magnetic tape recorder?

PART – B  (5 x 3 = 15 MARKS)

11.	What is phantom loading in an energy meter?

12.	Equal resistances of 100 each are connected in each arm of the Wheatstone Bridge supplied by a 2V Battery Source. The galvanometer of negligible resistance connected to the bridge can sense as low current as 1 A.  What is the smallest value of resistance that can be measured?

13.	Define RMS voltage. Give the principle of operation of a True RMS voltmeter.

14.	Voltage E1 and E2 of same frequency are applied to the horizontal and vertical plates of a CRO. What is the shape of pattern obtained on the CRO?  Explain how it is formed.

15.	Bring out the major advantages of using LEDs in electronic displays.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With relevant diagrams and explanation, show how moving iron instruments can be used

to measure both AC and DC quantities.   									      (10)

b. 	Find the multiplying power of a shunt of 200  resistance used with a galvanometer of 			1000   resistance. Determine the value of shunt resistance to give a multiplying power of 		50.																		        (5)

(OR)

17.	Prove that the deflecting torque is proportional to the true power consumed by the circuit in case of an electrodynamometer wattmeter from the first principles.

18.	a. 	With a neat phasor diagram, show how inductance can be measured using Anderson 				bridge.									             								(8)

b. 	Derive an expression for inductance under balance condition for Hay bridge.     	     	(7)

(OR)

19.	a. 	What are the limitations of Wheatstone bridge?                                        			(3)

b.	Illustrate how Kelvin double bridge overcomes the limitations of Wheatstone bridge with a 		neat circuit.    	                                                                                   			  	(12)

[P.T.O]

20. 	a.	With a neat block diagram explain how the magnitude and phase angle of impedance 	can be measured using vector impedance meter.		 							(10)

b. 	With a simple circuit explain the operation of a differential voltmeter.				(5)

(OR)

21. 	What is Harmonic Distortion? With a neat diagram, explain the construction and working 	principle of a harmonic distortion analyzer.

22.	Explain the principle of operation of digital storage oscilloscope with a neat block diagram.

(OR)

23.	a. 	With relevant sketches, explain how frequency and phase measurement can be made using 		CRO.																		(10)

b. 	Write a brief note on dual beam oscilloscope.									(5)

24.	a. 	With a neat block diagram explain the working of a digital frequency meter.			(8)

b. 	Elaborate on the working of U-V recorder and mention its advantages and limitations.																						(7)

(OR)

25. 	What are the advantages of digital recording? Explain the different types of digital tape recorders.