Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: INSTRUMENTATION AND PROCESS CONTROL FOR FOOD  	  	 			  ENGINEERS									  	      		       Time: 3 hours

Subject Code: 11EI215		       				        		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Servomechanism is __________.

2.	The region in control valve characteristic has _________.

3.	An Actuator is _________.

4.	What is meant by forcing function?

5.	State Mason’s gain formula.

6.	RTD is also called as________.

7.	Buoyancy meter is based on the principle of ________.

8.	State the principle of strain gauge.

9.	Calomel is ________.

10.	List down the 2 electrodes present in pH measuring of Wheatstone bridge.

PART – B  (5 x 3 = 15 MARKS)

11.	State the difference between open loop system and closed loop system.

12.	Give a brief note on Intelligent control valve.

13.	Comment on various categories of pressure measurement.

14.	State the principle of an Acoustic meter.

15.	Write the difference between Dispersive photometer and Non-Dispersive photometer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With a simple block diagram explain process control in detail. 						(8)

b.	A closed loop servo system is represented by differential equationd2Cdt2+8dCdt=64e 	where C is the displacement of the shaft, r is the displacement input shaft and e=r-C. 	Determine: 	i.	Damping ratio	ii.	Damped natural frequency 				(7)

(OR)

17.	a.	Explain different types of standard functions with their response.					(7)

b.	Derive the time response of a first order system subjected to:

i.	Unit step input function.

ii.	Unit ramp input function.

iii.	Unit impulse input function.  												(8)

18.	Explain the generation of P, PI, and PID controller action using pneumatic control.

(OR)

19.	With a neat sketch explain the working principle of pneumatically operated control valve and spring diaphragm actuator.

[P.T.O]

20.	a.	Explain the rules for Block Diagram reduction techniques.							(7)

b.	Determine C(s)/R(s) for the following block diagram using block diagram reduction 	technique. 																	(8)

(OR)

21.	a.	Find C/R for the signal flow graph shown. 										(7)

b.	Comment on the stability of the system described by the following equation:		(8)

3s4+10s3+5s2+5s+1=0.

22.	Compare the working principle of capacitive pressure sensors and inductive pressure sensors with neat diagrams.

(OR)

23.	a.	Explain the construction and working of a well type RTD with a neat sketch. 		(8)

b.	Give a brief note on thermoelectric sensors. 										(7)

24. 	Conduct a suitable experiment to measure the density of liquids and gases.

(OR)

25.	a.	Explain any one electrochemical method to determine composition. 				(7)

b.	With a schematic diagram explain mass spectrometer. 							(8)