Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PROCESS ENGINEERING CALCULATIONS			       	       Time: 3 hours

Subject Code: 11FP202				       					            	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The unit of Energy is________________

2.	__________________ is defined as the number of mole of solute dissolved in 1 liter of 	solution.

3.	Write the P, V, T relationship at two different points

4.	Humidity is _____________ / _________________.

5.	Write the material balance equation for unsteady state.

6.	In extraction input oil seed + solvent = ______________ + _____________.

7.	Draw the process flow diagram for a dryer.

8.	Give the example of concentration by non thermal processing.

9.	Define Heat capacity.

10.	Define Enthalpy.

PART – B (5 x 3 = 15 MARKS)

11.	If air consists of 0.72 moles of O2 and 2.75 moles of N2 calculate the mole fraction of oxygen and nitrogen.

12.	Explain the van der Waal equation, Dew point and Dalton’s law.

13.	Explain the total mass balance and component mass balance with an example.

14.	For 50% moisture removal by drying, write the mass balance equation for a 200 kg input with 60kg moisture.

15.	Explain about sensible heat and latent heat.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the important physical properties of solutions in detail.

(OR)

17.	a.	Define atomic mass, mole and molar mass. 										(3)

b.	Convert 499 g CuSO4.5H2O into mole.											(5)

c.	Sodium chloride weighing 600 kg is mixed with 200 kg potassium chloride. Find the composition of the mixture in terms of mass percent and mole percent.				(7)

18.	a.	Write the kinetic theory of gases.												(8)

b.	A process requires 10 m3/s (V1) at 202.65 kPa (P1) absolute pressure and 20C (T1). Determine the rating of a compressor in m3/s at STP 0C (T) and 101.325 kPa (P) which must be used to supply air for this process.										(7)

(OR)

19.	a.	Define relative humidity, saturation humidity, percentage humidity and humid heat.

(4)

b.	Explain how the humidity chart will be read for a given dry bulb temperature and dew

point.																		(4)

[P.T.O]

c.	The dry bulb temperature and dew point of ambient air were found to be 302 K (29C)

and 291 K (18C) respectively. The parameter reads 100 kPa a, compute:			(7)

i.	The absolute molar humidity.

ii.	The absolute humidity.

iii.	The percent relative humidity.

iv.	Saturation humidity.

20.	In oil extraction from soya bean seed using hexane, the seed contained 18% oil and the final extract solution contained 40 % of oil and 90% of the total oil is extracted. Write the mass balance. The crushed bean solids underflow (Heavier stream) retained in addition half their weight of solution after each settling stage.

(OR)

21.	a.	Define Distillation.															(2)

b.	A vapour at 411 K (138C) and standard atmospheric pressure, containing 0.72 mole fraction of benzene and 0.28 mole fraction toluene serves as a feed to a fractionating column in which it is separated into a distillate containing 0.995 mole fraction benzene and bottoms with 0.97 mole fraction toluene. For a 100 kmol, compute the overall and component material balance for benzene and toluene.								(13)

22.	a.	Convert 80% m.c. (wet basis) into dry basis (db).									(3)

b.	How much weight reduction would result when a material is dried from 80% moisture to 50 % moisture?																(12)

(OR)

23.	a.	How is evaporation different from drying?										(3)

b.	An evaporator has a rated evaporation capacity of 500 kg/h of water. Calculate the rate of production of juice concentrate containing 45% total solids from a raw juice containing 12% solids.																	(12)

24.	a.	Discuss the sensible heat changes in gases at constant pressure.					(8)

b.	Calculate the heat required to raise the temperature of a 4.535 kg roast from 4.44C to 65.55C. Express this energy in 	a.	Jolues	b.	Watt hours. Specific Heat Cavg = 3182 J / kgK.																	(7)

(OR)

25.	a.	Define internal energy and specific heat.

b.	Calculate the amount of water that must be supplied to a heat exchanger that cools 100 kg/h of tomato paste from 90C to 20C. The tomato paste contains 40 % solids. The increase in water temperature should not exceed 10C while passing through the heat exchanger. Specific heat of water = 4187 J/kgK and that of tomato paste = 2846.76 J/kgK.