Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PRINCIPLES OF FLUID MECHANICS AND HEAT TRANSFER														       							       Time: 3 hours

Subject Code: 11FP204				       						     	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define surface tension.

2.	What is an orifice meter?

3.	State the zeroth law of thermodynamics.

4.	Write the difference between refrigerator and heat pump.

5.	Define dryness fraction.

6.	What is a water tube boiler?

7.	State Fourier’s law for conduction heat transfer.

8.	What is critical thickness?

9.	Write the unit for convection coefficient.

10.	State Stefan Boltzmann’s law of radiation.

PART – B (5 x 3 = 15 MARKS)

11.	Find the surface tension in a soap bubble of 40 mm diameter when the inside pressure is 2.5 N/m2 above atmospheric pressure.

12.	Prove that R = CP - CV

13.	What are the specific enthalpy, entropy and specific volume of a dry steam with pressure of 20 bar?

14.	Explain thermal resistance for heat transfer through a cylinder of inner and outer radii of R1 and R2 with thermal conductivity of K.

15.	What is the difference between conduction and radiation heat transfer?

PART – C (5 x 15 = 75 MARKS)

16.	The inlet and throat diameters of a horizontal venturimeter are 30 cm and 10 cm respectively. The liquid flowing through the meter is water. The pressure intensity at inlet is 13.734 N/cm2 while the vacuum pressure head at the throat is 37 cm of mercury. Find the rate of flow. Assume that 4 % of the differential head is lost between the inlet and throat. Find also the value of Cd for the venturimeter.

(OR)

17.	Derive the expression for Euler’s equation of motion and hence derive the Bernoulli’s equation.

18.	A quantity of air occupying a volume of 1 m3 at 4 bar and 150ºC is allowed to expand isentropically to 1 bar. Its enthalpy is then raised by 70 kJ by heating at constant pressure. What is the total work done during the processes? If the processes are to be replaced by a reversible polytropic expansion which will result in the same final state being reached what index of expansion is required? Will the work done be greater or lesser than the original processes?

(OR)

[P.T.O]

19.	A heat pump working on a reversed Carnot cycle takes in energy from a reservoir at 3 ºC and delivers it to another reservoir where temperature is 77 ºC. The heat pump derives power for its operation from a reversible engine operating within higher and lower temperature limits of  1077 ºC and 77 ºC. For 100 kJ/sec of energy supplied to the reservoir at 77 ºC estimate the energy taken from the reservoir at 77 ºC.

20.	A steam turbine expands steam reversibly and adiabatically from 40 bar, 300 ºC to 0.12 bar, 50 ºC at turbine exit. Determine the work output per kg of steam. Also find the condition of steam at the exit of turbine.

(OR)

21.	With a neat sketch, explain the construction and working of a Fire tube boiler.

22.	Derive the expression for overall thermal conductance for a composite slab with three different materials of unit m2 area of cross section having thickness of L1, L2 and L3 and with thermal conductivities of K1, K2 and K3 respectively.

(OR)

23.	A pipe having outside diameter of 15 cm is covered with 5 cm layer of insulation having thermal conductivity of 0.035 W/m K. The pipe is 40 m long. The inside and outside surface temperatures are 320 ºC and 15 ºC respectively. Neglecting axial heat transfer,

a.	Calculate the radial conduction heat transfer in W.

b.	What thickness of insulation is needed to reduce the heat loss to 1400 W.

24.	Define the following:

a.	Convection heat transfer.			b.	Natural convection.

c.	Forced convection.   				d.	Dimensionless numbers.

e.	Reynolds number. 				f.	Convection heat transfer coefficient.

g.	Boundary layer thickness.

(OR)

25.	Two parallel opposed infinite black planes are maintained at 300 ºC and 400 ºC respectively. Find the net heat transfer between the planes. If the temperature difference is doubled by increasing the temperature of the plane with 400 ºC to 500 ºC, by what factor is net heat exchange increased?