Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: UNIT OPERATIONS IN FOOD PROCESSING – I		       	       Time: 3 hours

Subject Code: 11FP208			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define specific gravity.

2. 	What is Biot number?

3. 	Define partial vapour pressure.

4. 	Define distillation

5. 	What is enthalpy?

6. 	Define boiling point elevation

7. 	What is super saturation?

8. 	What is crystallization?

9. 	Give two application of extraction.

10. 	Define leaching.

PART – B (5 x 3 = 15 MARKS)

11.	Discuss steady and unsteady state.

12. 	Explain heating and cooling requirements.

13. 	Explain the vapor recompression.

14. 	Discuss about basis of solid liquid extraction.

15. 	Explain supercritical fluid extraction.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain briefly the conservation of mass and energy.	 							(5)

b.	Discuss briefly discontinuous and semi continuous process involving in food processing.

(10)

(OR)

17.	a.	Explain in detail the dimensionless ratio.	 									(5)

b.	Milk is flowing through a full pipe whose diameter is known to be 1.8 cm. The only  measure available is a tank calibrated in cubic feet, and it is found that it takes 1 hour to fill 12.4 ft3. What is the velocity of flow of the liquid in the pipe?                                 	(10)

18.	a.	Using a neat diagram explain in brief the continuous fractionation column.	  		(10)

b.	Describe briefly about flash distillation.	 										(5)

(OR)

19.	a.	What is plat efficiency? Explain briefly the types of plate efficiency.	 			(5)

b.	A batch of crude pentane contains 15 mol percent n – butane and 85 percent n – Pentane. If a simple batch distillation at atmospheric pressure is used to remove 90 percent of the butane, how much pentane would be removed? What would be the composition of the remaining liquid?                                                  									(10)

20.	a.	With the help of a neat sketch explain the climbing film tubular evaporator.	 		(10)

b.	Discuss single and multiple effect operation.                                                         		(5)

(OR)

[P.T.O]

21.	a.	An evaporator is continuously fed with 2500 kg per hour of solution, which has 10% of   	NaCl, 10% NaOH and remaining water. During evaporation, water is removed and NaCl 	precipitates as crystals or is removed. The thick liquor leaving has 50% NaOH , 5% NaCl  	and rest water. Find weight of salt precipated, water evaporated and thick liquor leaving.                                                         																				(10)

b.	Explain plate evaporators.                                                         						(5)

22. 	a.	Discuss briefly about primary nucleation.                                                               		(5)

b.	Describe about continuous crystallizer with a neat sketch.                        			(10)

(OR)

23.	a.	Explain in detail about gas – liquid equilibria.                                                             	(5)

b.	In crystallization process, 2500 kg of Na2SO4 crystallizes. The mother liquor leaving the process contains 20% Na2SO4 and during the process, 2.5% of feed evaporates as water vapor. If the feed has 40% Na2SO4 find the weight of feed and mother liquor.		(10)

24.	a.	Describe about the oleoresins and essential oils.                                                        	(5)

b.	Write briefly about the spray and packed extraction towers with a neat sketch. 		(10)

(OR)

25.	a.	With a neat sketch explain in detail the moving bed leaching.                                 	(10)                                                              	b.	Discuss in detail the ideal stages in counter current leaching.                        			(5)