Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MASS TRANSFER PROCESSES IN FOOD ENGINEERING      Time: 3 hours

Subject Code: 11FP301		         						              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain in detail about how the number of ideal plates in a distillation column will be found out by using McCabe-Thiele method.

2.	a.	Explain about the dimensionless equation and consistent units.

b.	Give a note on the dimensional equation with a suitable example.

(OR)

3.	Using only exact definitions and standards, calculate factors for converting:

a.	Newtons to pounds force.		b.	British thermal units to IT calories.

c.	Atmosphere to pounds force per square inch.

4.	Explain in detail about multiple effect evaporation with a neat diagram.

(OR)

5.	a.	Discuss the principle, construction, working of a falling film evaporator with a neat sketch.

b.	Explain about flash distillation with a suitable diagram.

6.	A triple effect evaporator is used to concentrate 1000 kg of aqueous solution from a concentration of 20 % solute to 80% solute. Assuming an equal amount of vaporization in each effect calculate the composition and weight of the solution entering the second and third effects.

(OR)

7.	Isopropyl alcohol and water can be separated by extraction with ethylene tetrachloride (C2Cl4). 100 kg of solution containing 30% (weight) isopropyl alcohol and the rest of water is mixed with the solvent ethylene tetrachloride. After extraction, the raffinate phase analyzed 71% water, 28.1% isopropyl alcohol and 0.9% ethylene tetrachloride. The extract phase analyzed 94% ethylene tetrachloride, 5.2% isopropyl alcohol and the rest water. Calculate the following:

a.	The amount of solvent.		b.	The quantities of raffinate and extraction phases.

c.	The percent extraction of isopropyl alcohol.

8.	Write short notes on liquid- liquid extraction with a suitable example.

(OR)

9.	Explain in detail about the super critical extraction system and phase equilibrium for CO2 and its uses.