Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF HEAT EXCHANGERS AND PRESSURE VESSELS FOR FOOD PROCESSING											       Time: 3 hours

Subject Code: 11ME205				       				          	   	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write the LMTD and AMTD equations.

2.	The unit of overall heat transfer coefficient “U” is _______________.

3.	Whether corrosive liquid should be taken inside or outside the tubes of a heat exchanger?

4.	What is the use of baffles in heat exchangers?

5.	Mention the Cooling Tower types.

6.	Write the pumping power calculation equation.

7.	Give an example of a recuperative heat exchanger.

8.	The annular space between the vessels in steam jackted heat exchanger is _______ mm.

9.	The purpose of evaporation is _____________.

10.	Give an example of a vertical type evaporator.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the fouling factor in heat exchangers.

12.	Explain the tube side heat transfer coefficient equation.

13.	Write the pressure drop in a shell side heat exchanger.

14.	Explain about Air preheaters.

15.	Mention some criteria for selecting an evaporator.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the effectiveness () calculation equation in heat exchanger for parallel flow.

(OR)

17.	a.	Explain the classification of heat exchangers by construction.						(10)

b.	A counter flow shell and tube heat exchanger is used to heat water at a rate of m = 0.8 kg/s 	from Ti = 30C to To = 80C with hot oil entering at 120C and leaving at 85C. The 	overall heat transfer coefficient U = 125 W/m2.C. Calculate heat transfer area required.

(5)

18.	a.	Explain the design consideration of a shell and tube heat exchanger.				(8)

b.	The Reynolds number of a water flow in heat exchanger is 10,300 and the Prandtl number 	is 2.22. Calculate the inside heat transfer coefficient ‘hi’ by using the relationship. Nusselt 	Number = 0.023 Re0.8 Pr0.3.													(7)

(OR)

19.	a.	Explain the baffles in heat exchanger like types, spacing and tube arrangement. 		(8)

b.	Explain the overall heat transfer coefficient ‘U0’ based on the outer surface with an ID of Di = 2.5 cm and an OD of D0 = 3.34 cm. k = 54 W/mC for the following flow and fouling conditions:

hi = 1800 W/m2C	h0 = 1250 W/m2 C 	Fi = Fo = 0.00018 m2C / W			(7)

[P.T.O]

20.	a.	Explain the construction and working of a cooling tower.							(8)

b.	Explain the double pipe heat exchanger and Enthalpy balances in heat exchanger and 	condenser.																	(7)

(OR)

21.	a.	How does a contact type condenser work? Explain.								(10)

b.	Explain the design procedure for pressure drop calculations in tube side and shell side heat 	exchanger.																	(5)

22.	a.	Discuss the recuperative and regenerative heat exchangers.						(10)

b.	Explain the high and low temperature applications of heat exchangers.				(5)

(OR)

23.	a.	Describe the jacketed batch reactor heating.										(10)

b.	Explain the dimensionless number relationship to calculate film coefficient ‘hb’. 		(5)

24.	Elaborate on the working of rising film and forced circulation evaporators.

(OR)

25.	Explain the single and multiple effect evaporators.