# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: ADVANCED FLUID MECHANICS & COMPUTATIONAL FLUID 				  DYNAMICS												 Time: 3 hours

# Subject Code: 11ME306			                 						 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Starting from the fundamentals derive the Navier Stokes  equations of motion in 	Cartesian co-ordinates.					        							(10)

b.	Derive the energy equation for a two dimensional heat and fluid flow problem for a 	steady state condition.														(10)

2. 	Explain Eulerian and Lagrangian methods of describing fluid motion.

(OR)

3. 	a.	Derive the Eueler’s equation for three dimensions flow.						(8)

b.	Given velocity field v=10x2yi +15xyj + (25t-3xy) k , find the acceleration of a fluid 	particle at a point (1, 2, -1) at time t=0.5.									(12)

4.	a.	A source of strength 120 m3/s/m is located at the origin and is subjected to a uniform 	velocity of 60m/s in the X direction. Locate the stagnation point and calculate the 	velocity at (1, 1). 	What is the value of the stream function on the half-body? Why is the 	width of the half –body on the Y-axis and what is its maximum width.	       		(10)

b.	The two dimensional stream function for a flow is Ψ=9+3x-4y+7xy. Find the velocity 	potential	.															       (10)

(OR)

5. 	a.	A source and sink each strength 25m3/s located on X axis 1m on either of the origin are 	exposed to a uniform stream of fluid which has a velocity of 40 m/s parallel to x-axis. 	Find the length and width of the Rankine body.					       			(10)

b.	Derive an expression for stream and velocity potential for doublet.		       		(10)

6.	a.	Explain Joukowshi transformation.						       					(12)

b.	Write short notes on Rankine body.						         				(8)

(OR)

7. 	a.	What is boundary layer? Explain with a sketch the development of boundary layer over a 	smooth flat plate.								       							(10)

b.	A cargo truck of width 3.3 m, height 3.5 m and length 14m travels at 90 kmph in air of 	kinematics viscosity 0.138 square centimeter per sec and density 1.196 kg/m3. Determine 	the power required to overcome the air resistance over the top and sides of the trunk. 	Find the boundary layer thickness at the rear end and the shear stress at mid-length of the 	truck. Assume that the front end of the truck is rough enough to make the entire 	boundary layer turbulent.							        					(10)

8. 	With a suitable example, explain the Crank-Nicolson method of solving a transient problem.

(OR)

9. 	a.	Discuss the advantages of direct and iterative methods of solving a CFD problem.																					(10)

b.	Solve ∂2u∂x2−2=0,when0<x<1,subjectedtou0=0∧u1=0, using finite 	difference method solution procedure. Also verify the results with exact solution. Find 	the value of u at x=0.4.								      					(10)