Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED THERMODYNAMICS AND HEAT TRANSFER   Time: 3 hours

Subject Code: 11ME307			       						             	      Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	One kg of ice at -5oC is exposed to the atmosphere which is at 20 oC. The ice melts and 	comes into thermal equilibrium with atmosphere.		a.	determine the entropy increase 	of the universe. 		b. What is the minimum amount of work necessary to convert the 	water back into ice at -5oC? Cp of ice is 2.093kJ/kg K and the latent heat of fusion of ice is 	333.3kJ/kg.																	(14)

b.	Air is flowing steadily in an insulated duct. The pressure and temperature measurements of the air at two stations A and B are given below. Establish the direction of the flow of air in the duct. Assume that for air, specific heat Cp is constant at 1.005 kJ/kg K, h=CpT, and      v/t =0.287/p, where p, v, and T are pressure (kPa), volume (m3/kg) and temperature (K) respectively.

Station A                              	  Station B

Pressure                           130 kPa                                   100 kPa

Temperature                     50 oC                                        13 oC  					(6)

(OR)

2.	Derive an expression for availability energy balance equation for:

a.	Closed system.

b.	Steady flow system.

3.	Show that for van der Waals’ gas:

a.	∂cv∂vT=0.

b.	(S2−S1)T=Rlnv2 −bv1−b.

c.	T(v-b)R/Cv = constant, for an isentropic.

d.	Cp−Cv=T1−2a(v−b)2RTv3 .

e.	(h2-h1)T=(p2v2-p1v1)+a1v1−1v2.

(OR)

4.	A mixture of ideal gases consists of 3 kg of nitrogen and 5 kg of carbon dioxide at a pressure of 	300 kPa and temperature of 20oC. Find: 		a.	the mole fraction of each constituent,

b.	the equivalent molecular weight of the mixture,		c.	the equivalent gas constant of 		the mixture,		d.	the partial pressures and the partial volumes, 	e. the volume and 	density of the mixture,	f. 	the Cp and Cv of the mixture.

If the mixture is heated at constant volume to 40oC, find the changes in internal energy, 	enthalpy and entropy of the mixture. Find the changes in internal energy, enthalpy and entropy 	of the mixture if the heating is done at constant pressure. Take γ for CO2 and N2 to be 1.286 and 	1.4 respectively.

[P.T.O]

5.	Octane (C8H18) is burned with dry air. The volumetric analysis of the products on a dry basis are CO2 = 10.02%, O2 = 5.62%, CO = 0.88%,  N2 = 83.48%. Determine :

a.	The air- fuel ratio.

b.	The percentage of theoretical air used.

c.	The amount of H2O that condenses as the products are cooled to 25o C at 100 kPa.

(OR)

6.	a.	Using the chemical equilibrium of ideal-gas mixture and the Gibbs function data, 	determine the equilibrium constant Kp for the dissociation process N2→ 2N at 25o C.	(10)

b.	Determine the temperature at which 10 percentage of diatomic hydrogen (H2) dissociates 			into monatomic hydrogen (H) at a pressure of 10 atm.								(10)

7.	Water at atmospheric pressure and saturation temperature is boiled in a 25 cm diameter, electrically heated, mechanically polished, stainless-steel pan. The heated surface of the pan is maintained at a uniform temperature Tw=116oC. Calculate:

a.	the surface heat flux.

b.	the rate of evaporation from the pan.

c.	the peak heat flux.

(OR)

8.	A heat exchanger is to be designed to cool mh=8.7 kg/s an ethyl alcohol solution (cph=3840J/(kg. oC  )) from T1=75oC  to T2=45oC  with cooling water [cpc=4180 J/(kg. oC)] entering the tube side at t1=15oC at a rate of mc=9.6 kg/s. The overall heat transfer coefficient based on the outer tube surface is U0=500 W/(m2 oC  ). Calculate the heat transfer area for each of the following arrangements:

a.	Parallel flow, shell and tube.

b.	Counter flow, shell and tube.

c.	One shell pass and two tube pass.

d.	Cross-flow, both fluids unmixed.

9.	Compulsory:

a.	Explain any two types of enhancement of heat transfer techniques.					(10)

b.	Detail the different types of measurement of errors.								(10)