Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ANALOG ELECTRONICS CIRCUITS I  				      	       Time: 3 hours

Subject Code: 11MT202			       					        	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why is a voltage divider bias commonly used in amplifier circuits?

2.	What is the maximum allowed gate current in JFET?

3.	Define CMRR.

4.	Write the Hybrid parameters equation for a transistor amplifier.

5.	Draw the low frequency simplified h-parameter model of an amplifier with an unbypassed emitter resistor.

6.	Mention the coupling methods used for coupling in multistage amplifiers.

7.	Write the maximum power handled by the class C power amplifier.

8.	Define inter modulation distortion.

9.	Define transformer utilization factor of a rectifier circuit.

10.	What is meant by second breakdown in a transistor?

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the value Rs of the source resistor, required to self-bias a p-channel JFET with IDSS = 25 mA and VP = 15 V. The gate-to-source voltage VGS is to be 5V.

12.	Compare the important specifications of CB, CC and CE amplifiers.

13.	State the reasons why the performance of an n-channel FET is better than a p-channel FET.

14.	What is meant by cross over distortion and how can it be eliminated?

15.	Mention the factors which define the safety operating area (SOA) of a transistor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List out the different types of bias compensation techniques.                               		(3)

b.	Design a voltage divider bias network using a supply of 24 V, β=110, ICQ=4 mA, and

VCEQ=8 V. Assume Ve=1/8 Vcc.                                                                           		(12)

(OR)

17.	a.	Write briefly about thermistor compensation technique.			     				(5)

b.	With a relevant circuit explain the self bias circuit using n-channel JFET.     			(10)

18.	Explain with a circuit diagram the boot strapped Darlington emitter follower. Also derive its

input impedance.

(OR)

19.	Discuss in detail the principle of a two stage RC coupled amplifier and determine the       	overall current gain, if the current gain in each stage is Ai.

20.	Explain the operation of a high frequency common source FET amplifier with a neat diagram. Derive the expression for:

a.	voltage gain. 						b.	input admittance.

c.	input capacitance. 					d.	output admittance.

(OR)

[P.T.O]

21.	a.	Draw the Hybrid π equivalent circuit of a BJT. 									(3)

b.	Using hybrid π model, draw the high frequency equivalent circuit of a CE amplifier and 			derive for higher cut-off frequencies.                                                                       	(12)

22.	With a neat circuit diagram explain the operation of a class B push pull amplifier and also      	discuss its merits and demerits.

(OR)

23.	Explain how does a transformer coupled Class-A power amplifier differ from a direct coupled Class-A power amplifier? Compare the two with respect to:

a.	power dissipated in the transistor		b.	conversion efficiency.

24.	Discuss in detail how output voltage can be regulated with respect to line variations 	and load 	variations using SMPS.

(OR)

25.	Write short notes on:																(7+8)

a.	Voltage Multiplier.					b.	Power Control using SCR.