Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRONIC DEVICES   								       Time: 3 hours

Subject Code: 11MT203					       		    			       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is an intrinsic semiconductor?

2.	State Mass Action Law.

3.	The forward dynamic resistance of a junction diode varies __________ as the forward current.

4.	Write the diode current equation.

5.	What is tunneling?

6.	Why is a Zener diode used as a voltage regulator?

7.	The transit time of the current carries through the channel of a FET decides its ______ characteristics.

8.	Define β of a transistor.

9.	What is DIAC?

10.	What is the effect of negative gate current on normal SCR?

PART – B  (5 x 3 = 15 MARKS)

11.	Derive an expression for the force experienced by an electron placed in a magnetic field.

12.	What is Hall co-efficient? What are the applications of Hall Effect?

13.	Define transition and diffusion capacitance.

14.	Draw the Eber’s Moll model for a PNP transistor and give equations for emitter current and collector current.

15.	What is negative resistance in UJT?  Explain its physical significance.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the motion of electrons in perpendicular Electric and Magnetic field.

(OR)

17.	a.	For a semi conductor material Nc=1.5x108cm-3, Nv=1.3x1019cm-3 and EG=1.43 eV at 	T=300 K.  Find the intrinsic carrier concentration of the semiconductor and also determine 	the effective masses of electron and holes.										(10)

b.	Draw the energy band structure of an insulator, semiconductor and metal.			(5)

18.	Draw the energy band diagram of a P-N Junction diode under open circuit condition and explain how it will be modified if the P-N junction is forward biased.

(OR)

19.	Briefly explain the following:

a.	Drift current			b.	Diffusion current

20.	Illustrate and explain the V-I characteristics of PN junction in detail.

(OR)

21.	Explain in detail the Avalanche and Zener breakdown mechanism of a Zener diode.

22.	Explain the input and output characteristics of CE configuration.

(OR)

[P.T.O]

23.	Discuss the construction and working principle of N-channel FET with a neat diagram.

24.	Draw the two transistor model of an SCR and explain the principles of operation of SCR.

(OR)

25.	Draw the equivalent circuit of a UJT and explain its principle of operation.