Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL ELECTRONICS									       Time: 3 hours

Subject Code: 11MT210			       				             			       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Convert (53.625)10 to binary.

2.	Convert (10110)2 to Gray code.

3.	What are universal gates?

4.	Suggest a logic circuit that can add more than 1-bit.

5.	Write the characteristic equation of a D-flip flop.

6.	How many flip-flops are required to design a modulo-7-counter?

7.	In a state diagram, state is represented by ______.

8.	What is critical race?

9.	What is a ROM decoder circuit made up of?

10.	Which is the non-saturated logic?

PART – B (5 x 3 = 15 MARKS)

11.	If X=1000011; Y=1010100, perform X-Y using 2’s complement method.

12.	Realize X-OR using NAND gates.

13.	With characteristic table explain the operation of a T-flip flop.

14.	Compare Mealy model and Moore model.

15.	What is fan-out?

PART – C (5 x 15 = 75 MARKS)

16.	Simplify the following using k-map method:

a. 	y = m1+m5+m10+m11+m12+m13+m15.											(7)

b. 	y = ∑ m(0,2,3,6,7) + ∑ d(8,10,11,15).											(8)

(OR)

17.	Find the minimum sum of products for the Boolean expression:

f = ∑(1,2,3,7,8,9,10,11,14,15) using Quine - Mcclausky method.

18.	a. 	Design a Full subtractor. 														(7)

b. 	Implement the following Boolean function a using multiplexer:

F(x,y,z) = ∑(1,2,6,7)															(8)

(OR)

19.	Design a BCD to excess 3 code converter.

20.	Design a 3 bit synchronous binary counter using D-flip-flops.

(OR)

21.	Design a Mod-10 Counter.

[P.T.O]

22.	Draw the state diagram for the given state table:

| Present State | Next State X = 0 X = 1 | Output X = 0 X = 1 |
| --- | --- | --- |
| a b c d e f g h | f b d c f e g a d c f b g h g a | - 0 - 0 - 0 - 0 - 0 - 1 - 1 1 0 |

Reduce the above state table and draw the state diagram for the reduced state table.

(OR)

23.	Design a 4-bit asynchronous binary counter.

24.	Implement the following Boolean functions using PAL:

a.	Y3 = A'BC'D + A'BCD' + A'BCD + ABCD'

b.	Y2 = A'BCD' + A'BCD + AB'CD

c.	Y1 = A'BC' + A'BC + AC + ABC'

d.	Y0 = ABCD

(OR)

25.	Compare the performance of various logic families.