Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMPUTER NETWORKS								       Time: 3 hours

Subject Code: 11MT228			       				             		      	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	How does Manchester encoding help in achieving better synchronization?

2.	What are the key benefits of a layered network?

3.	What is crosstalk?

4.	A channel has B = 10 MHz. Determine the channel capacity for a signal-to-noise ratio of 60 	dB.

5.	How is the inefficiency of Stop-and-Wait protocol overcome in sliding window protocol?

6.	Explain how a virtual connection is defined in ATM.

7.	How is performance improved in CSMA/CD protocol compared to CSMA protocol?

8.	What are the advantages of DNS?

9.	In what situation is flooding most appropriate?

10.	What are the two basic mechanisms of congestion control?

PART – B (5 x 3 = 15 MARKS)

11.	Make a critical comparison of communication switching techniques with the help of an event timing diagram.

12.	A TCP connection with 65,535 bytes windows is sent over a 1 Gbps channel.  The channel has a round trip delay of 20 m secs. Find:

a.	The maximum achievable throughput.	b.	Line efficiency

13.	Find the shortest path from A to D for the network shown.

14.	One way of detecting errors is to transmit data as a block of ‘n’ rows of ‘k’ bits per row and adding parity bits to each row and each column. The lower-right corner is a parity bit that checks its row and its column. Will this scheme detect:

a.	Single errors? 	b.	Double errors? 	c.	Triple errors?

15.	Discuss the difference between multiplexing and de-multiplexing techniques with relevant examples.

PART – C (5 x 15 = 75 MARKS)

16.	Discuss in detail the protocol stack and its functionality. How does a TCP stack differ from an 	OSI stack?

[P.T.O]

(OR)

17.	Explain the internet backbone with an architecture supporting it.

18.	Write a TCP socket program to establish communication between client and server and perform file transfer.

(OR)

19.	Write a UDP socket program to establish communication between client and server and 	perform file transfer.

20.	With suitable illustrations, explain Selective Repeat sliding window protocol.  How is it better 	than Go-back-N protocol?

(OR)

21.	Draw the format of a TCP segment and briefly explain the function of each field.

22.	Compare the Datagram subnets and Virtual-circuit subnets. Explain in detail about Virtual circuits with an example.

(OR)

23.	Explain routing. Give detailed notes on:

a.	Shortest Path Routing		b.	Distance Vector Routing

24.	Discuss the Error correction and Error detection codes. Explain the Hamming code and Cyclic 	redundancy checks with examples.

(OR)

25.	Discuss in detail the concept of Network Management.