Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: INTRODUCTION TO COMPOSITE MATERIALS & STRUCTURES														      						       Time: 3 hours

Subject Code: 12AE227			       					         	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define composite materials.

2.	List down the various fiber materials.

3.	Define isotopic and anisotropic materials.

4.	Define micromechanics.

5.	Define macromechanics.

6.	Define symmetric and antisymmetric laminates.

7.	Define sandwich construction.

8.	List down the materials used for sandwich construction.

9.	Define open mould process.

10.	List down various types resins used in composite materials.

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the expressions for elastic constants in stress-strain relations in terms of material 	properties for isotropic and anisotropic materials.

12. 	Derive the expression for longitudinal modulus of elasticity of a lamina in terms of modulus of 	elasticity of fiber, modulus of elasticity of resin and volume rations of fiber and resin.

13.	Explain how lamina properties, modulus of elasticity and Poission’s ration are determined 	experimentally.

14.	Write the governing equation for a general laminate and simplify the same for isotropic 	material.

15. 	Explain how netting analysis is carried out composite structural analysis.

PART – C  (5 x 15 = 75 MARKS)

16. 	Show that the member elastic constants required to relate stress to strain is two for isotropic 	materials.

(OR)

17. 	a.	List down the various type of composite materials used and their advantage over 				conventional materials.														(5)

b.	List down the applications composite materials in various fields engineering.		(10)

18.	The material properties along the principal material directions (fiber direction, l and transverse direction t) are: Ee = 145 GPa, Et = 10.5 GPa. Get = 7 GPa. ⋎et = 0.3. Compute the stiffness coefficients of stress-strain relations for a lamina in which the fiber is oriented at an angle of 450.

(OR)

19. 	Derive the expressions for modulus of elasticity along  an arbitrary directions x (Ex) in terms of 	material properties with respect to principal material direction and fiber orientation angle Ɵ 	with the x – axis.

[P.T.O]

20. 	Derive the governing equation for the analysis of a general laminated plate.

(OR)

21. 	Compute the coefficients of laminate matrices A, B, and D for a cross-ply laminate 	(0/90). 	Thickness of each lamina is 0.125 mm. Ee = 140 GPa, Et = 10 GPa, Get = 5 GPa, ⋎et = 0.3.

22. 	 The properties of glass-epoxy angle ply lamina (600) are as follows.

Along the fiber tensile strength = 1280 MPa, Normal to the fiber tensile strength = 49MPa, 	Along two fiber compressive strength = 622 MPa, normal to the fiber compressive strength is 	245 MPa. Shear strength is 69Mpa. Modulus of elasticity along the fiber is 35 GPa and that 	along the direction of normal to the fiber is 7 GPa. Modulus of rigidity is 3GPa. ⋎e1  = 0.3. A 	tensile load 2 MPa is applied along x – direction and fiber is at angle 600 to x – axis. Check for 	safety of lamina using maximum stress theory, maximum strain theory and Tsai – Hill theory.

(OR)

23. 	Derive the expressions for flexural rigidity for a general sandwich construction and simplify

the same for a sandwich construction with facings made of the same material and equal 	thickness.

24. 	 Explain with a neat sketch the details of one open mould process and one closed mould 	process.

(OR)

25. 	 List down the structural components fabricated by filament winding and explain in detail with 	a neat sketch the filament winding process.