Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: INTRODUCTION TO NON DESTRUCTIVE TESTING		       Time: 3 hours

Subject Code: 12AE229			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give one advantage of Non Destructive Testing over destructive testing.

2.	What is Level 1 certification in NDT?

3.	What is the basic principle of dye penetrant testing?

4.	What is the basic requirement for a material to be tested by magnetic particle inspection?

5.	How do the following casting defects appear in a radiograph?

i.	blow holes 	ii.	 cracks.

6.	Name the two commonly used radioactive sources in industrial gamma ray radiography.

7.	What is the function of a coupling agent in ultrasonic testing? Name two coupling agents       	used in ultrasonic testing.

8.	Give two applications where eddy current testing is extensively used.

9.	Differentiate between thermal and quantum detectors used in the thermal imaging technique.

10.	What are the main components of an Acoustic Emission Testing equipment.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the common things agreed upon between the manufacturer and inspection agency, prior to non-destructive testing?

12.	What are the different detecting media used in Magnetic Particle Testing?

13.	What are the factors that influence the choice of film in radiography?

14.	What is the commonly reported information in an eddy current test report?

15.	Briefly explain how corrosion can be detected by infrared imaging?

PART – C  (5 x 15 = 75 MARKS)

16.	What is NDT? Discuss briefly the important techniques used in Non Destructive Evaluation of materials and components.

(OR)

17.	What are the different levels in NDT certification? Discuss in detail.

18.	Discuss in detail the steps in dye penetrant testing with relevant sketches.

(OR)

19.	Explain in detail how magnetic particle testing is done.

20.	Discuss in detail how x-ray radiographic testing is done. Give the advantages and 	disadvantages of this technique.

(OR)

21.	Discuss how the different types of defects that occur in weldments can be identified using radiography with simple sketches.

[P.T.O]

22.	Explain how ultrasonic testing is done. Give the advantages, disadvantages and applications of this technique.

(OR)

23.	Discuss briefly the different types of probes used in eddy current testing.

24.	Briefly discuss the different types of detectors used in thermal imaging technique.

(OR)

25.	What is Acoustic Emission Technique? Explain the equipment used for this technique. Give the advantages, limitations and application of acoustic emission.