# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: PROCESS EQUIPMENT DESIGN				  	     		 Time: 3 hours

# Subject Code: 12BT307							         	  		 Maximum Marks: 100

Answer ALL questions

a. Any missing data may be suitably assumed

b. Chemical Engineering Hand book or any two chemical engineering books may be permitted

PART A ( 1 X  50 = 50 marks)

1. 	A shell and tube heat exchanger cools the organic liquid from 80oC to 40oC at the rate of 2700 kg/hr. Cooling water is entering at 30oC and leaving at 35oC. The following data is available;

| Properties | Organic liquid | Water |
| --- | --- | --- |
| Heat capacity Cp J/kgk | 1400 | 4180 |
| Thermal conductivity W/mk | 0.096 | 0.66 |
| Viscosity N-s/m2 x103 | 0.4 | 0.72 |
| Density kg/m3 | 780 | 1000 |

Water is circulated on tube side while the organic liquid is circulated on shell side. Tubes of 25 mm O.D and 1.5 mm wall thickness arranged on 31mm square pitch are selected. Tube length is 3m. Assume:

a. 	Single pass on shell side.

b. 	Four pass on tube side.

c. 	Overall heat transfer coefficient = 250 – 750 W/m2k.

d. 	Co-current flow condition.

e. 	Friction factor Ft = 0.92.

g. 	Fouling resistance on shell side = 0.0002 m2k/W.

h. 	Fouling resistance on tube side = 0.0004 m2k/W.

Analyze the assumed data and suggest required dimensions for the shell and tube heat exchanger.																	      (50)

(OR)

2. 	7927 kg/hr of monochloro benzene is to be produced by reaction of benzene with chlorine. Chlorine is carried out at 30-40oC, by reacting 0.6 moles of chlorine per mole of benzene. A continuous reactor may be preferred at the expense of some monochlorobenzene being converted to dichlorobenzene. The industrial process uses upto 0.5% FeCl3 as catalyst at 80oC and 1.0 kg/cm2. To minimize dichlorobenzene formation, the conversion is restricted to 45% using a feed ratio of 0.45 moles of chlorine per mole of benzene. Under these conditions dichlorination is 8 times and trichlorination is 240 times slower than monochlorination. Suggest a suitable design of a Continuous stirred tank reactor.

The yield of monochloro benzene is 95.3%.

C6H6 + Cl2  C6H5Cl + HCl

Density of liquid benzene = 810kg/m3.

Viscosity of benzene at 80oC = 0.31 x10-3 N-s/m2.

Surface tension = 20 dynes/cm.

Initial concentration of benzene CAo = 810 kg/m3.

CB* = 73.57 kg/m3.

kL  = 1.345 m/hr;     k2 = 0.374 m3/m-hr.

βf = 0.728 ;    CBout = 73.5 kg/m3.												      (50)

[P.T.O]

PART –B (1 X 50 = 50 marks)

3. 	A double drum dryer is to be designed for drying a paste with a capacity of 110 kg/hr. The dryer is heated with indirect steam available at atmospheric pressure (100oC).

The following data is available:

The temperature of paste = 20oC.

Initial moisture content of paste = 70% (wet basis).

Final moisture content of paste = 10%.

Heat transfer from the condensing steam to steam wall = 8500 W/m2k.

Heat capacity of the paste material = 3400 J/kgk.

Thermal conductivity of the paste material = 0.8 W/mk.

The thickness of the layer material = 1 mm.

Thickness of iron drum wall = 10 mm.

Thermal conductivity of iron drum = 45 W/mk.

Air is blown over the surface of the material at a velocity of 1.6 m/s.

Temperature of the air = 420C.

Relative humidity of air = 40%.

Latent heat of vaporization of water at atmospheric pressure = 2240 kJ/kg.

Maximum temperature of the outer surface of the material being dried = 80oC.

Vapour pressure of water at 80oC = 350 mmHg.

Partial pressure of water vapour in air at 42oC and relative humidity 40% is = 22 mmHg.

Rate of flow of moisture being evaporated can be estimated by the correlation:

G = 1.14 x 10-5 u0.8 (ΔP)

u- velocity of air flow over the surface m/s.										      (50)

(OR)

4. 	A Feed containing 50 mole% A and 50 mole% B is to be distilled in a fractionating column to get top product containing 99 mole% and bottom product containing 10 mole% A. The relative volatility of the binary mixture is 2.5. The reflux ratio of 2 kgmole per kgmole of product is used. The feed enters at its bubble point.

Estimate the number of plates required in rectifying section, stripping section and the total number of plates.																      (50)