Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC CIVIL ENGINEERING							      	       Time: 3 hours

Subject Code: 12CE101			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention any two disciplines in Civil Engineering related to water resources.

2.	Write the expansion of EIA.

3.	Give any two types of surveys based on purpose.

4.	Give two types of loads acting on a structure.

5.	Write the standard size of a brick.

6.	Write the expansion for PCC and OPC.

7.	What are the components of a railway track?

8.	What do you mean by SH and NH?

9.	Mention any two purposes of dams.

10.	Give two examples of hydropower projects in India.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the typical responsibilities of civil engineers.

12.	Briefly explain the basic principles of surveying.

13.	What are the important requirements of stone as a construction material?

14.	Draw the cross-section of a highway and label the important components.

15.	Draw the layout of a municipal waste water treatment plant and label the important	components.

PART – C  (5 x 15 = 75 MARKS)

16.	Write short notes on the scope of civil engineering.

(OR)

17.	Explain the various functions of a civil engineer.

18.	The following perpendicular offsets were taken at 10 m intervals from a survey line to an irregular boundary line: 4, 8, 5, 8, 7, 4 and 3.  Calculate the area enclosed between the survey line and the boundary line by the Trapezoidal rule and the Simpson’s rule.

(OR)

19.	Find the moment of inertia through the censorial axis for the cross-section given below:

[P.T.O]

20.	Briefly explain the various properties of cement and concrete.

(OR)

21.	Write short notes on paint and emulsions.

22.	Draw the typical cross section of a permanent way and write the requirements of a permanent way.

(OR)

23.	Draw the general layout of a harbour and explain the features.

24.	Briefly describe the points to be considered in the selection of a site for a dam.

(OR)

25.	Write short notes on hydropower projects.