Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: INDUSTRIAL WASTE TREATMENT AND DISPOSAL	   	       Time: 3 hours

Subject Code: 12CE259			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by ‘Environmental Ethics’?

2.	Define ‘4R’ technology.

3.	Explain the objective of Equalization.

4.	What are the factors that influence the Coagulation process?

5.	List three chemicals used to remove phosphorus from wastewater.

6.	What are the typical wastewater characteristics of textile and apparel industries?

7.	Define metabolism.

8.	Define TOC.

9.	List out the waste products generated during cement manufacturing.

10.	What is spent wash and where is it generated?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between waste minimization, waste exchange and recycling.

12. 	Explain the chemical oxidation process in removal of heavy metals.

13.	Compare Aerobic and Anaerobic decomposition of organic matter.

14.	Draw the flow chart for fertilizer manufacturing process.

15. 	What are the effects of tanning industry on the environment?

PART – C  (5 x 15 = 75 MARKS)

16. 	Classify the possible contribution of Industrial waste that can cause pollution. Briefly discuss 	the effects and difficulties caused by it when discharged in natural streams.

(OR)

17. 	Explain in detail how Process Modification and by-product recovery reduce strength of waste

from industries.

18.	a.	Explain how neutralization of acid waste and alkaline waste is carried out in Industries.																					(8)

b.	Discuss the solid separation process of Industrial wastes.							(7)

(OR)

19. 	Explain in detail about Coagulation and Flocculation process.

20. 	Design a conventional activated sludge plant to treat domestic sewage with diffused air aeration

system with the following data: Population = 35000, average sewage flow = 180 lpcd, BOD

sewage = 220 mg/I, BOD removal primary tank = 30%, overall BOD reduction = 85%, FM ration = 0.35, MLSS = 2000 mg/I. Verify any two conditions for design limits.

(OR)

21. 	What is reverse osmosis? With a neat sketch explain the reverse osmosis method of 	desalination.

[P.T.O]

22. 	Describe the characteristics of the wastes produced by petroleum refinery industry and the 	possible treatment requirements.

(OR)

23. 	a.	Give a suitable treatment method to remove dissolved inorganic substances from industrial

wastewater.																	(8)

b.	Explain how metals are removed from Industrial waste effluents.					(7)

24. 	Brief on the sources and characteristics of wastewater from a Diary Industry and explain the 	treatment of diary waste with a flow diagram.

(OR)

25. 	Explain the process in tanning industry with a neat flow chart. State the ill effects of tannery 	effluents on the sewer and on treatment plants when it is discharged without treatment.