# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: APPLIED ELASTICITY AND PLASTICITY				 Time: 3 hours

# Subject Code: 12CE302							                	 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A large rectangular flat plate with a small circular hole of radius ‘a’ in the centre is subjected to uniform tensile stress of magnitude ‘s’ per unit area in the x direction. Derive the expression for the stress distribution in the plate in the vicinity of the hole. Sketch approximately the shape of the stress diagram.

2.	Derive the equilibrium equations in Cartesian co-ordinates for a stressed body in three dimensions.

(OR)

3.	The state of strain at a point is given by

εx=15x10-4 		εy= 45x10-4 		εz= 30x10-4

τxy = 25x10-5    	τyz= 15x10-5         τxz = –30x10-5. Determine the strain invariants and the 	principal strains.

4.	Investigate what problem of plane stress is solved by the stress function:φ=3F4Cxy−xy33C2ρ2y2.

(OR)

5.	A beam of narrow rectangular cross section is subjected to uniformly distributed load of q over the entire span. Assuming suitable stress function, derive expressions for stresses and displacements, if the beam is simply supported.

6.	Derive the expressions for shear stresses for a hollow rectangular section. Find the shear stresses in a hollow rectangular section of size 230mm x 180mm with wall thickness of 15mm subjected to a torque of 10kN-m.

(OR)

7.	A shaft with an elliptical cross section is subjected to a twisting moment ‘T’. Find the expression for maximum shear stress anywhere in the cross section.

8.	a.	A rectangular beam 8 cm wide and 10 cm deep is 50 cm long. It is simply supported at 	ends carries a concentrated load at the centre. If ‘P’ is the load increased so that the outer 	2 cm of the depth of beam yields plastically, determine the value of ‘P’. Yield stress of 	material is 250MN/m2. Assume linear stress- strain curve.

b.	Explain the following 		i.	 Flow rule 	ii.	Tresca’s theory

iii. Von- Mises criterion

(OR)

9.	a.	Discuss the Elasto-plastic problems giving suitable examples.

b.	Describe the stress- strain representation through Westergard stress space.