Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED DESIGN OF STEEL STRUCTURES		      	       Time: 3 hours

Subject Code: 12CE304		         						              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

( assume relevant missing data )

( IS 800-2007, IS 1893-2002, IS 802 and Steel Tables are permitted )

1.	Design a stiffened seat connection for a beam ISWB 600 @ 1337 N/m transmitting 300 kN to the web of a stanchion ISHB 250 @ 547 N/m. Assume that the steel as Fe 410 and the bolts as grade 4.6

(OR)

2.	Design an unstiffened welded seat connection using shop weld to connect ISMB 400 @ 616 N/m to transmit a factored load of 250 kN to the flange of a stanchion ISHB 300 @ 588 N/m. Steel grade is Fe 410.

3.	Calculate the safe uniformly distributed load per meter length which an ISMB 550 @ 1037 N/m can carry over an effective span of 4m. The ends of the beam are restrained against torsion but not against lateral bending.

(OR)

4	Design a beam column for a non sway intermediate column for a multistory industrial building frame with flexible joints of 5 m high for the following data.

Factored axial load 			= 425 kN

Factored BM	– at the top	 	= 32.5 kNm

– at the bottom 	= 60 kNm

5.	Compulsory:

The roof trusses for a factory building are spaced at 5 m c/c and the purlins are spaced at           2 m c/c. the pitch of the roof truss is ¼ and the span of the truss is 10 m. the dead load from the roofing material is 1 kN/m2, live load 2 kN/m2, wind load 1.2 kN/m2. Design the purlin with an angle section.

6	The figure shows the section of a beam, laterally supported at an interval of 1.5 m. Assuming Cb as 1.0, fy = 235 N/mm2, E = 2x105 N/mm2, calculate the allowable bending moment for the section.

[P.T.O]

(OR)

7	A 55m microwave antenna lattice tower is to be built at Coimbatore. The terrain at the site location is nearly a level ground with terrain of category 2. It has to carry a 3m diameter hemispherical antenna disc at the top. The necessary data is given below:

Width at top of the tower 				= 3.5m

Weight of antenna disc and fixtures 	= 10 kN

Weight of platform at top 				= 0.85 kN/m2

Weight of railing at top    				= 0.3kN/m2

Weight of ladder and the cage 			= 0.6kN/m

Weight of miscellaneous items			= 2.5kN

Wind loads are as per IS 875 part III. Select a suitable truss configuration and determine the maximum compressive force and tension in the leg at the base and also the maximum transverse shear force at base.

8.	A two span continuous beam ABC of uniform cross section is loaded with ultimate loads as shown in figure.  Determine the required plastic moment of resistance.

(OR)

9  	Find the value of Mp for the portal frame loaded upto collapse as shown in the figure.