Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: STRUCTURAL DYNAMICS							       	       Time: 3 hours

Subject Code: 12CE305				        				              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Determine the natural frequencies and mode shapes of the given system shown in the figure.

2.	a. 	A system vibrating with a natural frequency of 6Hz starts with an amplitude of 2cm and an 	initial velocity of 25cm/s.  Determine the natural period, amplitude of vibration, maximum 	velocity, maximum acceleration and phase angle.

b.	A vibrating system consisting of a weight of 1000kN and a spring of stiffness of 80kN/m is viscously damped so that the ratio of two consecutive amplitudes is 1 to 0.85. Determine the logarithmic decrement, natural frequency, damping ratio, damping coefficient and damped natural frequency.

(OR)

3.	a.  	An  SDOF system consists of a mass of 20kg, a spring of stiffness 2200N/m and a dashpot 	with a damping coefficient of 60Ns/m is subjected to a harmonic excitation of F = 200 	sin 5t N.  Determine the natural frequency, damping coefficient, frequency ratio and steady 	state response.

b.	A machine weighing 1000kg is supported on foundation by spring mounting. The machine 		is subjected to a harmonic excitation force of amplitude 490N and frequency of 180rpm. 			Determine the maximum force transmitted to the foundation if the total stiffness of the 			spring is 2000kN/m.

4.	Derive the Fourier series expression for the given periodic loading function and write the expression for the steady state response of an SDOF system.

F(t) =  Fo     0 ≤  t  ≤ Tp/2

F(t)  =  0     Tp/2 ≤   t ≤   Tp

(OR)

5.	a.	Explain how general dynamic loading is represented by Duhamel’s integral.

b. 	A suddenly applied force F1 was applied to an SDOF system. Compute the maximum 	dynamic load factor by using Duhamel’s integral.

6.	Determine the normal modes of transverse vibration of a simply supported beam of length L and uniform cross section.

(OR)

[P.T.O]

7.	a. 	Find approximate fundamental frequency of a uniform cantilever beam by taking the shape 	function Ψ (x) = (x/L)2.

b.	Determine the first two natural frequencies of a uniform cantilever beam by Rayleigh-Ritz method.

8.	a.	Define Orthogonality and give the relation for the orthogonal condition between normal			modes.

b.	Explain Stodola’s method of computation of Eigen values and Eigen vectors.

(OR)

9.	a. 	Explain the concept of Shear building.

b. 	Derive an expression for computing the dynamic response of a MDOF system using Wilson’s θ method.