# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: FINITE ELEMENT METHODS IN ENGINEERING 		  Time: 3 hours

# Subject Code: 12CE308				         		        			  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory

a.	Show the equivalence of Rayleigh- Ritz and Galerkin methods. As an example consider the second order differential equationd2ydx2−f(x)=0; in Ω0<x<L with boundary condition y = 0 at x = 0 and x = L.					    						(12)

b.	Derive the governing equations for lateral vibration of a beam loaded as shown in the 	figure below 	using variational approach.							    		  (8)

2.	A rod of diameter 10mm, length 200mm has nodal displacements due to axial loads as 1.2mm and 2.8mm. The position of rod is shown in the figure below. Calculate:

a.	The displacement at a point Q on the rod. Also find			b.	Strain

c.	Stress 				d.	The strain energy for the rod. Take E = 210Gpa.

(OR)

3.	a.	Derive the interpolation functions for a four node iso-parametric quadrilateral element as 	shown in the figure below.					    								(8)

[P.T.O]

b.	An iso-parametric parent element is shown in figure (a), and a corresponding iso-	parametric distorted element is shown in figure (b). Discuss the transformation that 	relates partial derivatives in the original x, y coordinates to the generalized ,  	coordinates.										    						(12)

4.	a.	Evaluate the following integral using n = 2 Gaussian Quadration:

y=0ax=0b(xydxdy) = a2b24.												(10)

b.	Drive a local stiffness matrix for heat transfer using shape functions for a four node 	quadrilaterial element.								    						(10)

(OR)

5.	Derive the shape functions for axisymmetric triangular element and obtain the stress-strain relationship matrix.

6.	a.	Explain the various plate theory and finite element solutions for plate element.	(12)

b.	Distinguish between finite element and finite strip methods.			    		(8)

(OR)

7.	a.	Discuss in detail about thin and thick shell theories.				    			(10)

b.	Explain the types of boundary conditions in degenerated and axisymmetric shell element.										    										(10)

8.	a.	Describe preprocessing, processing and post processing using any finite element 	software and write the step-by-step procedure.						    		(12)

b.	Discuss the discretization error and how to evaluate it.			    			(8)

(OR)

9.	a.	Explain the various engineering applications of finite element method and mention the 	field variable in each application.						   					(10)

b.	Discuss about auto and adaptive mesh generation techniques.			    		(10)