Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PRESTRESSED CONCRETE STRUCTURES			       	       Time: 3 hours

Subject Code: 12CE309				         					       	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

Use of relevant IS codes is permitted(IS 1343(1980),IS 3370(PART III))

Assume data, if necessary

1.	A post-tensioned PSC beam, 200mm wide and 300mm deep, is prestressed with wires of area 320mm2 located at a constant eccentricity of 50mm and carrying an initial stress of 1000 N/mm2. The span of the beam is 8m. Calculate the percentage loss of stress in the wires, using the following data: Es= 210 kN/mm2 and Ec= 35kN/mm2.	Relxation of stress in steel is 5% of the initial stress. Shrinkage of concrete is 200 x 10-6. Creep coefficient =1.6, Anchorage slip = 1mm and frictional coefficient for wave effect is 0.0015 per m.

(OR)

2.	a.	What are the advantages of prestressed concrete over reinforced concrete? 			(5)

b.	A simply supported prestressed concrete beam of c/s 400 x 600 mm covers a span of 10m. 	It is subjected to an udl of 30 kN/m in addition to its self weight and is prestressed with a 	prestressing force of 1740 kN with a prestressing force of parabolic profile. The cable is 	anchored at the center of gravity of the cross section at support section and has a dip of 160 	mm at the mid span. Analyze the beam at mid span cross section using the philosophy of 	stress concept.																(15)

3.	a.	What are the different types of flexural failures in PSC members?					(4)

b.	A post-tensioned concrete T-Beam with unbounded tendons is made up of a flange 300mm wide by 150 mm thick and the width of rib = 150 mm. The effective depth of the section=320 mm. The beam is prestressed by 24 wires of 5mm diameter with an initial stress in steel as 1650 N/mm2. Loss ratio is 0.8 and if the cube strength of concrete is 50 N/mm2, estimate the flexural strength of the section using IS code.					(16)

(OR)

4.	a.	Explain type I, II and III structures in prestressed concrete.						(4)

b.	A post tensioned concrete beam of rectangular section 250 mm wide and 500mm deep has a span of 12.50m and carries a superimposed load of 8.5 kN/m. The tendon is provided with a parabolic profile with a central dip of 140 mm and with no eccentricity at the ends. The effective prestressing force in the tendon is 750 kN. Determine

i.	The principal stresses at the supports.

ii.	The principal stresses at the supports without prestress.

Take weight of concrete = 25 kN/m³.											(16)

5.	a.	Define transmission length and give its expression.								(4)

b.	The end block of a pre – stressed concrete beam 100 mm wide and 200 mm deep, supports an eccentric prestressing force of 100 kN, the line of action of which coincides with the bottom kern of the section.  The depth of the anchor plate is 50mm.  Estimate the magnitude and position of the principle tensile stress on a horizontal plane passing through the centre of the anchorage plate and also calculate Bursting Tensile force by Guyon’s method and compare that value with IS method.									(16)

[P.T.O]

(OR)

6.	A precast pretensioned unit of rectangular section of size of 120 mm x 240 mm is used a part of a composite beam to span 6m.This unit is prestressed by tendons with their centroid coinciding with the bottom kern point. The initial force in the tendon is 240 kN. The loss of prestress may be assumed to be 15%.The unit is incorporated as the web of a composite beam by casting a slab of flange width of 480 mm and the thickness of 40 mm. On the top of the precast unit the composite beam supports a live load 4.0 kN/m. Calculate the resultant stresses developed in the precast and cast in-situ concrete assuming the pretensioned unit as unpropped while casting the in-situ slab. The ratio of modulus of elasticity between the precast unit and cast in-situ slab is 1.25.

7.    a.	Give the various methods of achieving continuity in prestressed concrete construction.

(4)

b.	A two-span continuous prestressed concrete beam ABC (AB = BC=15m) has a uniform cross section with a width of 250 mm and a depth of 600 mm. A cable carrying an effective prestressing force of 500 kN is parallel to the axis of the beam and located at an eccentricity of 200mm.

a.	Determine the secondary and resultant moment developed at mid support section B.

b.	If the beam supports an imposed load of 2.4 kN/m, calculate the resultant stresses at 	the top and bottom of the beam at B.

c.	Locate the resultant line of thrust through the beam AB. Take weight of 						concrete=24kN/m³														(16)

(OR)

8. 	a.	What are the advantages of continuous members in PSC structures?				(4)

b.	A continuous beam ABC (AB=BC=20m) with an overall depth of 1m is prestressed by a 	continuous cable carrying a force of 300 kN. The cable profile is parabolic between the 	supports, with zero eccentricity at ends A & C. The cable has an eccentricity of 100 mm 	towards the soffit at mid span section and 200 mm towards the top fiber at mid support 	section. Calculate the reactions developed at the supports due to prestress and show that 	the 	cable is concordant.														(16)

9. 	Compulsory:

Design a non-cylinder prestressed concrete pipe of internal diameter 500 mm to withstand a working pressure of 1 N/mm2. High-tensile wires of 2 mm diameter stressed to 1200 N/mm2 at transfer are available for use. Permissible maximum and minimum stresses in concrete at transfer and working loads are 13.5 and 0.8 N/mm2 (compression) respectively. Loss ratio=0.8, Es = 210 kN/mm2 and Ec = 35 kN/mm2	. Calculate:

- The minimum thickness of concrete for the pipe.

- Number of turns of wire per meter length of the pipe.

- The test pressure required to produce a tensile stress of 0.7 N/mm2 in the concrete when applied immediately after tensioning.

- The winding stress in the steel.