# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: DESIGN OF FOUNDATIONS                 					  Time: 3 hours

# Subject Code: 12CE310				                 				 	  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- Bring out the steps carried out by a foundation engineer to arrive at the suitable 	foundation for a given structure in a given site.   								(8)

b.	Design a friction pile group and find out the settlement of the same for the following 	data:

i.	Load including the weight of pile cap 				= 2500 KN.

ii.	The depth of uniform clay underlain by rock 		= 18 m.

iii.	Average unconfined compressive strength of clay 	= 60 KN/m2

iv.	Liquid limit of the clay 							= 55 %

v.	Factor of safety against shear 						= 2.5

Assume that the load to be transferred at two-third of the length of pile.   		(12)

2.	a.	With the help of the sketches, list out the difference between “Free Earth Support” and 	“Fixed Earth Support” with respect to anchored bulk head.  				    	(8)

b.	Determine the depth of embedment of sheet piling which is given in the figure below. 																				(12)

(OR)

3.	a.	Bring out the different types of coffer dams with sketches. List the uses of each coffer

dam. 															     		(10)

b.	How do you check the stability of a coffer dam? 						  	       (10)

4.	a.	Explain “Free Vibration”, “Degree of Freedom” and “Resonance in machine 				foundation”.  												             		(10)

b.	Explain the various vibration isolation techniques in machine foundation.     	(10)

(OR)

5.   a.	Bring out the design criteria for the design of foundation for Impact machines and 			reciprocating machines.   													(12)

b.	A machine weighing 400 KN is mounted on a concrete foundation block resting on a soil

layer. The plan area of the foundation block is 25 m2. Its weight is 1000 KN. Assume that the system is subjected to unbalanced force in vertical direction. The coefficient of the uniform compression for the soil is 105 KN/m3. Calculate the natural frequency of the machine foundation if:   i.	weight is kept constant and foundation area is doubled.

ii.	Area is kept constant and weight is doubled.         						(8)

[P.T.O]

6.	a.	Explain the various tests performed by the Geo technical Engineers to identify the 	expansive soil.															(10)

b.	Suggest the various methods for laying the foundation in expensive soil. 	 	(10)

(OR)

7.   	a.	Sketch the under ream pile with reinforcement and determine the load carrying capacity 	of under reamed piles in clayey and sandy soil.  								(10)

b.	Determine the load carrying capacity of a 40 m long single under reamed pile of  50 cm 	stem diameter. Average cohesion value both within the strata of pile depth and below the 	toe is 100 KN/m2.														(10)

8.  	a.	Explain the vibro floatation techniques.  									(10)

b.	Discuss the method of installation of stone columns with a neat sketch.  		(10)

(OR)

9.  	What is reinforced earth? Bring out the details of different materials which are used in reinforced earth. What are the advantages in reinforced earth structures?