# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: SEISMIC DESIGN OF STRUCTURES  					 Time: 3 hours

# Subject Code: 12CE311				         		     			        Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

Use of 1893:2002, 13920:1993, IS4326:1993, IS800:1987 is permitted

1.	Compulsory:

An RC moment resisting frame building with three floors is located at Delhi. The floor 	height is 3m. The live load can be taken as 3kN/m2. The columns are 250 x 300 mm and 	beams 250 x 275 mm. The slab is 120 mm thick. The building has 3 bays in x and 3 bays in y 	direction with a bay width of 3 m. The soil below is of rock strata. Use M20 and Fe415 	steel. Assume the building is for a school.  Determine the total base shear as per 	IS1893(Part 	I) : 2002 and 	distribute along the height of the building.

2.	Write  brief notes on the following:

a.	Elastic Rebound theory.													(6)

b.	Seismic waves.															(8)

c.	Magnitude and Intensity.													(6)

(OR)

3.	“Earthquakes don’t kill people, manmade structures kill”. Substantiate this quote with reference to the Bhuj earthquake.

4.	Explain the ductility considerations for an RC moment resisting framed structure as per IS13920:1993.

(OR)

5.	What is meant by Capacity based design. Write the step by step procedure for Capacity based design.

6.	Explain the modes of failure of infilled frames. Explain the equivalent strut model for analysis of an infill frame.

(OR)

7.	What is a Braced frame? What are its types? Explain the procedure for calculating the total lateral drift of a doubly braced frame.

8.	Write brief notes on the following:

a.	Base Isolation technique.													(10)

b.	Push over Analysis.														(10)

(OR)

9.	Explain the various retrofitting methods and techniques.