# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: 	ADVANCED DESIGN OF BRIDGES	        				Time : 3 hours

# Subject Code: 	12CE313		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

Assume missing data suitably

(Use of relevant IS&IRC codes are permitted)

1. Analyse the RCC slab bridge for a National Highway using the following data:

Carriage width			=	7.5m

Foot path				=	1m on either side

Clear span				=	6m

Wearing coat				=	100 mm

Width of bearing			=	0.5 m

Materials				=	M30 and Fe 415

Loading					=	IRC class AA tracked

(OR)

2.	Explain the failure of RCC Bridges indicating the possible reasons for such collapse.

3. 	Design a post tensioned pre-stressed concrete slab bridge deck for a National highway using the following data. Also check the stresses at top and bottom fiber.

Live load Moment		= 	200kNm.

M40 grade concrete and 7mm dia high tensile wires with an ultimate tensile strength of    1500 N/mm2 housed in cables with 12 wires. For supplementary reinforcement, adopt Fe415 grade steel. Compressive strength at transfer is 35 N/mm2 and loss ratio is 0.8.

(OR)

4. 	Write short notes on the following:

a.	Anchorage of tendons and Grouting of tendons.								(15)

b.	Advantages of Prestressed concrete bridges.									 (5)

5.	Compulsory:

Design the plate girder bridge for the following data:

Effective span of girder      					= 25 m

Dead load 									= 10 kN/m

Equivalent total live load for bending moment

calculation per track							= 2727 kN

Equivalent total live load  for shear calculation

per track										= 2927kN

Design flange plate, web plate, stiffeners and connections between the elements

6. 	Verify the stability of the pier shown in the figure below.

Top width of pier 								=2m

Height of pier									=12m

High flood level									=2m below bearing level

Side batten										=1 in 12

Span of bridge									=16m

Loading											=IRC class AA

Size of the girder									=300x1500 mm (3nos)

[P.T.O]

1

12

2m

12m

(OR)

7. 	The foundation for sub structure of a bridge consists of 16 piles to carry a total load of    10000 kN. The piles are spaced at 2m c/c. They are driven through soft ground to a hard stratum available at a depth of 10m. Design the pile foundation using M30 and Fe 415.

8.	Give in detail the various methods adopted in maintenance of the bridges.

(OR)

9.	Write notes on Bearings in detail with design specifications.