Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: GROUND WATER HYDROLOGY						       	       Time: 3 hours

Subject Code: 12CE335				         				              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

(Semi-log sheet and Error function table are to be supplied)

1.	Compulsory:

a.	Differentiate between the following terms:										(6)			i.	Confined and unconfined aquifers.

ii. 	Aquifuge and aquiclude.

b. 	Explain the role of groundwater in hydrological cycle.							(7)

c.	Write short notes on groundwater budget.										(7)

2.	a.	A field test for permeability consists in observing the time required for a tracer to travel 		between two observation wells. A tracer was found to take 10 hours to travel between two 	wells 50 m apart when the difference in the water-surface elevation in them was 0.5 m. The 	mean particle size of the aquifer was 2 mm and the porosity of the medium is 0.3. If 	v=0.01 cm2/sec, estimate:

i.	The coefficient of permeability and intrinsic permeability of the aquifer.

ii.	The Reynolds number of the flow. 					          				    	(10)

b.	An aquifer has a specific yield of 0.19. During a drought period, the following average declines in the water table were noted:

Area    Size (km2)   Decline (m)

A            15                    2.34

B            7.5                   1.22

C            18.3                 0.76

D            22.5                 3.44

E            9.44                 1.89

F             22.7                 0.35

What was the total volume of water represented by the decline in the water table?   (10)

(OR)

3.	a.	A confined aquifer has 3 layers. The bottom layer consists of coarse sand of thickness 4 m, 	having hydraulic conductivity of 0.01 cm/sec. The middle layer consists of fine sand of 	thickness 3 m, having hydraulic conductivity of 0.002 cm/sec. The top layer consists of 	gravel of 4 m having hydraulic conductivity of 2 cm/sec. Calculate the equivalent 	hydraulic conductivity of the confined aquifer  if:

i.	the flow is along the stratification.

ii.	the flow is normal to the stratification.   	                            						  (12)

b.	Define the following terms:						      						           (8)

i.	Porosity and Void Ratio.

ii.	Hydraulic head and fluid potential.

iii.	Darcy’s law.

[P.T.O]

4.	a. 	Write in detail about the procedure to estimate ‘S’ and ‘T’ using Theis method.           (10)

b. 	List down the assumptions in Thiem equation.                                                                 (4)

c. 	A fully penetrating artesian well of 0.3 m diameter was pumped with a constant rate of 	1800 m3/day under a steady state condition. If the transmissivity of the aquifer is 0.145 	m2/min and the drawdown observed in the well is 8.92 m, find the radius of influence of 	the well.																		(6)

(OR)

5.	a. 	Using Jacob method, obtain S and T for a given aquifer. The time-drawdown data is given 	in the table for which the observation well is 12.3 m from the pumping well.             	(12)

| Time in (min) | Drawdown s (m) |
| --- | --- |
| 1 | 0.24 |
| 2 | 0.24 |
| 3 | 0.28 |
| 4 | 0.32 |
| 6 | 0.37 |
| 8 | 0.41 |
| 10 | 0.45 |
| 14 | 0.49 |
| 18 | 0.51 |
| 22 | 0.53 |
| 28 | 0.54 |
| 35 | 0.57 |
| 45 | 0.64 |
| 55 | 0.65 |
| 65 | 0.68 |
| 80 | 0.69 |
| 100 | 0.74 |
| 120 | 0.76 |

b.	A fully penetrating well of diameter 0.3 m is located in an unconfined aquifer of saturated 		depth 45 m. If the drawdown in the well is 15 m for the discharge of 1200 m3/day and the 		radius of influence is 300 m, compute hydraulic conductivity.			     			(4)

c. 	Define the following terms:

i. 	Cone of depression and radius of influence.					     				(2)

ii. 	Specific capacity of the well.						     						(2)

6.	a. 	Explain the following terms:

i. 	Advection.															      (4)

ii. 	Pulse and slug input.												             (4)

b. 	A saline solution with a concentration of 1823 mg/L is introduced into a 2-m-long sand column in which the pores are initially filled with distilled water. If the solution drains through the column at an average linear velocity of 1.43 m/day and the dynamic dispersivity of the sand column is 15 cm, what would the concentration of the effluent be 0.7 day after flow begins?									            				(12)

(OR)

7.	a. 	Explain in detail about physical processes (dispersion and diffusion) that control the

contaminant transport in subsurface media.										(8)    b.	Define the following terms:

i. 	Retardation.										      						(2)

ii. 	Tracer.																	(2)

c.	Write in detail about the types of reactions that can change the fate of contaminants in the 	subsurface media.															(8)

[P.T.O]

8. 	a. 	Describe the different equipments and methods used to measure groundwater level in a 	well.																		(10)

b.	What do you understand by artificial recharge and explain in detail about any two artificial 	recharge techniques								                 					(10)

(OR)

9.  	a.	Discuss in detail about any three types of wells. 									(12)

b.	Write short notes on:

i.	Land subsidence.									    						(4)

ii.	Seawater intrusion.									    					(4)