# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: COMMUNICATION AND SWITCHING TECHNIQUES	 Time: 3 hours

# Subject Code: 12CS334					                 			 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain the various layers of the ISO-OSI model.								(10)

b.	An amplifier has an input resistance of 600 Ω and a resistive load of 75 Ω. When it has an r.m.s input voltage of 100 mV, the r.m.s. output current is 20 mA. Find the gain in dB.        																		(5)

c.	What are the different networks required for various services of a Public Telecommunications Operator (PTO). 										(5)

2.	a.	Briefly explain message switching and circuit switching. 						(5)

b.	Describe the functions of a switching system.								(9)

c.	On average, during the busy hour, a company makes 120 outgoing calls of average duration of 2 minutes. It receives 200 incoming calls of average duration 3 minutes. Find a.	the outgoing traffic 	b.	the incoming traffic 	c.	the total traffic.  		(6)

(OR)

3.	a.	A PBX has three operators on duty and receives 400 calls during the busy hour. 	Incoming calls enter a queue and are dealt with, in order of arrival. The average time 	taken by an operator to handle a call is 18 seconds. Call arrivals are Poissonian and 	operator service times have a negative exponential distribution.

i.	What percentage of calls have to wait for an operator to answer them?

ii.	What is the average delay, for all calls and for those which encounter delay?

iii.	What percentage of calls are delayed for more than 30 seconds?			(12)

b.	Derive an expression for the probability of delay.  							(8)

4.	a.	Explain the principle of grading.											(10)

b.	Design a grading for connecting 20 trunks to switches having ten outlets.		(10)

(OR)

5.	a.	With a neat sketch, explain fully interconnected three-stage switching network. 	(10)

b.	Design a three-stage network for 100 incoming trunks and 400 outgoing trunks.	(10)

6.	a.	With a simplified flow diagram, explain the state transition for a local call.		(10)

b.	Explain stored- program control.										 	(10)

(OR)

7.	a.	Describe the phantom circuit for DC signaling over four-wire audio-frequency.   (10)

b.	Discuss the following terms applied to inter-register signalling: en-bloc signalling, 			overlap signalling, compelled signalling, link-by-link signalling, end-to-end signalling. 																				(10)

8.	a.	With a typical packet format, explain the principles of Packet switching.		(12)

[P.T.O]

b.	A terminal is connected to a computer by a datalink transmitting at 2.4 kbit/s in each direction. On average, the terminal user issues a request to the computer once per minute. The average length of a user’s message is ten 8-bit characters and the average length of the computer’s reply is 100 8-bit characters. The time taken by the computer to process the information and the propagation time of the link are negligible. Find:

a.	The average delay before the terminal has received a complete reply.

b.	The occupancy of the data channel. 									(8)

(OR)

9.	a.	Explain the principles and architecture of ISDN network.					   	(12)

b.	Describe in detail about Automatic Alternative Routing (AAR).  				(8)