# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: DIGITAL IMAGE PROCESSING              					 Time: 3 hours

# Subject Code: 12CS339				         		        		 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.  Compulsory:

a.	What are the elements of digital image processing system? Explain the function of each 	element in detail. 										    					(10)

b.	Explain the following gray level transformation techniques in detail:

i.	Image negative 			ii.	Thresholding

iii.	Contrast Stretching 	 	iv.	Gray level slicing					   		(10)

2.  	a.	Compute the 2D DFT of the 4 x 4 gray scale image given below:

f[m,n] = 1111111111111111												(10)

b.	With a suitable illustration, explain how image smoothing is performed in the frequency

domain.																	(10)

(OR)

3.  	a.	With a suitable illustration, explain how image sharpening is performed in the frequency 	domain.																	(10)

b.	With a neat block diagram, explain the phenomenon of homomorphic filtering.	(10)

4.  	a.	Give four valid differences between lossless and lossy image compression technique. 	Also obtain the Huffman code for the word ‘COMMITTEE’.					(10)

b.	With a suitable block diagram explain transform based image compression scheme. 																					(10)

(OR)

5.  	a.	List four  advantages of using wavelets in image compression. Also mention the types of

redundancy present in images. What are the techniques that can be used to minimize the

redundancy present in the image?											(10)

b.	With a neat block diagram, explain wavelet based image compression scheme.	(10)

6.  	a.	Define the term ‘edge’ associated with respect to image. What are the techniques that can 	be employed to detect the edge in an image?  Explain any two techniques in detail.																				(10)

b.	With a suitable illustration, explain split and merge technique for image segmentation.																				(10)

(OR)

7.  	a.	Explain the region growing method of image segmentation. Why region growing 	method is considered as bottom-up approach?								(10)

b.	Explain how thresholding technique can be employed in the process of image 	segmentation.  Also give the Sobel mask to detect the horizontal and the vertical edges 	in an image.																(10)

[P.T.O]

8.  	a.	What are chain codes? How the chain codes can be used to represent the boundary of an 	image? 																	(10)

b.	Explain how relational descriptors can be used to represent the boundary of an image.																				(10)

(OR)

9.  	a.	With a suitable example, explain how Fourier descriptors can be used to represent image

boundary.																(10)

b.	What is polygonal approximation? Explain how polygonal approximation can be used 	for image boundary representation?											(10)