Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MOBILE COMMUNICATION							      	       Time: 3 hours

Subject Code: 12EC244			       				               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is an isotropic radiator?

2.	The space between the interference ranges is called _________.

3.	What is MSC?

4.	GEO satellites have a distance of almost __________ to the earth.

5.	Give the functions of main service channel.

6.	What is the main task of PHY management?

7.	Taking a packet out of the data part of another packet is called ___________.

8.	Define MANET.

9.	What is fast re-transmit?

10.	HTTP is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is uplink and downlink?

12.	Name the three subsystems of GSM.

13.	Draw the IEEE 802.11 protocol architecture and management.

14.	What is the purpose of DHCP? Name the entities of DHCP.

15.	List the advantages and disadvantages of selective transmission.

PART – C  (5 x 15 = 75 MARKS)

16.	What is spread spectrum? Explain about the FH techniques with the frequency spectrum      	and list out the uses of the FH.

(OR)

17.	Describe the Cellular system with three and seven cell clusters and discuss the advantages      	and disadvantages of cellular systems.

18.	Explain the functional architecture of a GPRS transmission plane protocol reference model.

(OR)

19.	With a neat sketch, explain the DECT system architecture reference model and draw the        	DECT multiplex and frame structure.

20.	Discuss the IEEE 802.11 protocol architecture and bridging.

(OR)

21.	a.	Write short notes on Hyper LAN.												(8)

b.	Compare Digital Audio Broadcastings and Digital Video Broadcastings with examples.		                                                     													(7)

22.	What are the ad-hoc networks? Explain about any two ad-hoc networks in detail.

(OR)

[P.T.O]

23.	a.	Illustrate packet delivery to and from the mobile node using one example.			(8)

b.	Give notes on the typical initialization of a DHCP client.							(7)

24.	a	Explain the several mechanisms of the transmission control protocol (TCP). 	    	(7)

b.	Compare indirect TCP, Snooping TCP and mobile TCP.  				   			(8)

(OR)

25.	Explain the components and interface of the WAP 1.x architecture.