Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CMOS DIGITAL INTEGRATED CIRCUIT DESIGN		       Time: 3 hours

Subject Code: 12EC302				         				              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- With relevant diagrams explain the working of CMOS Domino logic. Mention its advantages and disadvantages. 													(10)

- Design and implement a CMOS logic circuit whose outputs are complemented, while the input signal given is also in its true and complement form. Discuss the same with its clocked version.																(10)

2.	Using Y chart, discuss the various design domains and their levels of abstraction, giving examples at each level.

(OR)

3.	a.	Derive the generalized output voltage expression of a CMOS inverter, if the input voltage 	is 3.2V, while the maximum supply voltage is 5V.								(12)

b.	What are saturated loads that can be used in inverters? Discuss at least two of them.	(8)

4.	a.	A register that fits in a data path is 25 µm tall. A metal 2 clock line runs vertically to link 	all registers in an n-bit register. The register has 30 µm of 1 µm metal 1, 20 µm of 1 µm 	poly and 16 µm of 1 µm gate capacitance. Calculate the per-bit clock load and the load for 	a 16 bit register using the table below.											(6)

Typical 1 µm CMOS Capacitances

--------------------------------------------------------------------------------------------------------

Capacitor 		Capacitance 						Description

aF (10-18)/µm2

--------------------------------------------------------------------------------------------------------

Cp 					50						Poly to substrate - over field oxide

Cm1 				30 						Metal1 to substrate - over field oxide

Cm1p 				60 						Metal1 to poly

Cm1d 				60 						Metal1 to diffusion

Cm2 				20 						Metal2 to substrate - over field oxide

Cm2m1 				50 						Metal2 to metal1

Cm2p 				30 						Metal2 to poly

Cm2d 				30 						Metal2 to diffusion

Cgs 				1800 					Gate Capacitance

- Derive the analytical delay model that describes the CMOS inverters Switching characteristics.																(14)

(OR)

5.	Calculate the total power dissipated by a CMOS circuit, deriving individual power component equations. Also discuss how power can be minimized.

6.	Discuss the structured design approaches used in the design strategies of a VLSI chip.

(OR)

[P.T.O]

7.	a.	Identify and explain few self-test techniques and explain them with relevant diagrams.

(12)

b.	Write short notes on statistical fault analysis performed on any digital circuit. 		(8)

8.	Explain the working of an array multiplier and serial-parallel multipliers with neat diagrams.

(OR)

9.	Draw a generic RAM architecture with its support circuits. Explain the working.