# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: ANALYSIS AND DESIGN OF ANALOG INTEGRATED CIRCUITS																			 Time: 3 hours

# Subject Code: 12EC311							                 	 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain the causes for various sources of electronic noise. Explain how these noise sources are modelled in integrated circuit components.

2.	a.	Draw the small signal model of an n-channel MOS and explain the parasitic elements in 	it.  								   										(10)

b.	With a neat circuit diagram and a small-signal equivalent diagram, derive an expression 	for the transition frequency of a MOS transistor.                       					(10)

(OR)

3.	a.	Explain the various factors that results in MOS device voltage limitations. 		(10)

b.	A bipolar transistor has a short-circuit, common emitter current gain at 1 GHz of 8 with Ic=0.25 mA and 9 with Ic= 1 mA. Assuming that high-level injection effects are negligible, calculate Cje and τF , assuming both are constant. The measured value of Cµ is 10 pF.                                                                      							(10)

4.	a.	What are the advantages of class B output stage? With a circuit diagram, explain the 	characteristics of an all-NPN class B output stage.                          				(10)

b.	What are the limitations of common source output stage? Explain how it is overcome in 	combined common-drain, common source output stage.           					(10)

(OR)

5.	What is the need for temperature-insensitive biasing? With a circuit diagram explain the working of band-gap referenced bias circuits in CMOS technology.

6.	a.	Define slew rate. Draw the schematic of a two-stage MOS op-amp for slew rate 	calculations. Explain the procedure.                                                                 	(12)

b.	Describe the methods of improving slew rate in two-stage opamps.              		(8)

(OR)

7.	Compare zero-valued time-constants with short-circuit time-constants. Describe the analysis on the frequency behaviour of single stage transistor amplifier with short-circuit time constants.

8.	Draw the circuit of folded-cascode opamp with active cascodes gain enhancement auxillary amplifier. Explain its operation.

(OR)

9.	a.	What is current mirror? Draw the circuit of a simple MOS current mirror and explain its 	operation.                                                                                    					(10)

b.	Draw the circuit of a cascode current mirror using MOS transistor and explain its 	operation.                                                                                   					(10)