Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SEMICONDUCTOR DEVICES AND MODELING	             	       Time: 3 hours

Subject Code: 12EC314			       						              	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	A silicon pn junction at T = 300 K with zero applied bias has doping concentrations of Na = 5  1016 cm3, Nd = 5  1016 cm3, ni = 1. 5  1010 cm3. Determine Xn Xp , space charge width (W) and peak electric field (Emax) in the given pn junction.

(OR)

2.	a.	With necessary energy band diagrams and equation, explain the concept of Schottky

barrier diode and compare it with pn diode.										(10)

b.	Justify with equations that the total current in the pn junction is the sum of individual electron and hole currents.														(10)

3.	Compulsory:

a.	Imagine that you have been assigned a project to design a BJT transistor for the following

specifications:

- Should amplify the input signal		ii.	Should have high gain

Explain how to achieve these specifications.										(6)

b.	Justify that “hFE improvement in BJT is the reason for band-gap narrowing effect”.	(4)

c.	With necessary timing diagrams, explain the switching behavior of BJT.			(10)

4.	a.	Mention the significance of linking a model file to a circuit designed using any one of the

H-Spice tool.																	(4)

b.	Derive the current equations for Ebers-Moll model.								(16)

(OR)

5.	a.	Describe the impact of high injection effect and early effect in BJT transistor.		(10)

b.	Derive the current equation for Gummel-Poon model.								(10)

6.	a.	Consider an n-channel MOSFET with source and drain doping concentrations Nd = 	1019cm3. Assume a channel length of L = 1.2 µm and assume the source and body are at 	ground potential. Calculate the therotical punch-through voltage assuming the abrupt 	junction approximation.														(10)

b.	With a neat diagram and necessary graphs explain the current voltage relationship of MOSFET.																	(10)

(OR)

7.	a.	Discuss various scaling issues listed below for MOSFET in 22 nm technology.	 (10+10)

i.	ON current		ii.	OFF current		iii.	ON/OFF ratio	iv.	Gain

b.	Draw the structure of FAMOS and MESFET. List the differences of the two structures.

8.	a.	Explain in detail about the short channel effects in n-channel MOSFETs.			(10)

b.	Define velocity saturation and explain how velocity saturation becomes more prominent in short channel devices.															(10)

(OR)

9.	a.	Derive the relation between drain current and channel length and discuss the issues in 	channel length modulation.													(16)

b.	List the reasons for mobility variations in MOSFETs.								(4)