# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: SMART ANTENNAS	                    							 Time: 3 hours

# Subject Code: 12EC344				    		        			 	  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	With a neat block diagram of an antenna array system, explain the elements of smart antenna systems.

(OR)

2.	a.	i.	Find the maximum effective aperture of an antenna which operates at wavelength of 		λ=2m and has directivity D=100.

ii.	Determine the radiative resistance Rr of an antenna radiating 10 KW and 12 Amps.																					(4)

b.	Calculate the gain of an antenna with a circular aperture of diameter 3m at 5GHz.  	(8)

c.	Write short notes on Beam Steering and Digital Steering.							(8)

3.	Derive the array correlation matrix in terms of

a.	Steering Vector Representation 	b. 	Eigenvalue Decomposition      			(12+8)

(OR)

4.	Describe about the tapped delay line structure for broad band signals beam forming and derive the frequency response.

5.	Compulsory:

Draw the real and complex beam former structures and list out the implementation issues of a real time beam forming system.

6.	Explain in detail the different MUSIC algorithms of DOA estimation.

(OR)

7.	Discuss about how the signal parameters estimation via ESPRIT is a computationally efficient method of DOA estimation.

8.	The desired signal and K co-channel interferences are present in a Nakagami fading environment. All interferences have the same statistics with the fading parameter denoted by m, the same as the desired signal, and have equal mean power denoted by I .How will you measure the selection combiner performance using selection algorithms for the above environment?

(OR)

9.	Examine the performance of an optimal combiner when there are more co-channel interferences than the number of elements in the array in fading conditions.