Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC ELECTRICAL ENGINEERING					       Time: 3 hours

Subject Code: 12EE101			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The SI unit for power is _________.

2.	Ohm’s law is true for a ____________ conductor at constant temperature.

3.	The total number of lines of force in the magnetic field is called_________.

4.	Coupling coefficient between two magnetically coupled circuits is always ___________ to one.

5.	Form factor and peak factor for a sinusoidal wave are _____ and ______.

6.	Mention any one demerit of a Hydro power plant.

7.	List any two types of single phase I.M.

8.	_________ and _________ are the two main parts of a DC motor.

9.	Induction type energy meter is an indicating instrument.  State true or false.

10.	Number of two-way switches used in staircase wiring to control a single load is __________.

PART – B (5 x 3 = 15 MARKS)

11.	State Kirchoff’s current law and voltage law.

12.	State Faraday’s laws of Electromagnetic induction.

13.	An alternating voltage is given by the equation v=282.84 sin (377t+300). Find the:

a.		RMS value 		b.		Frequency

14.	Mention the applications of DC series, shunt and compound motors.

15.	Write down the types of wiring.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain the concept of parallel connection of resistances.							(7)

b.	A resistance R is connected in series with a parallel circuit comprising of two resistors 12 and 8 respectively.  The total power dissipated in the circuit is 70 W when the supplied voltage is 22 volts.  Calculate the value of R.										(8)

I

R = ?

12

8

22V

+

-

(OR)

17.	a.	A filament lamp is rated for 100 W, 110V. Find the value of resistance to be connected in

series with the lamp so that it can be operated on a 230V supply. What is the power loss in the resistor?																	(10)

b.	Give the voltage division equation in a series circuit, and the current division equation in a parallel circuit.																(5)

[P.T.O]

18. 	a.	Compare magnetic circuit and electric circuit.							 		(10)

b.	A square cross sectional magnet of side 4 cm has magnetic flux strength of 0.5milliWb. Determine magnetic flux density.												(5)

(OR)

19. 	a.	Write down the laws of electromagnetic induction and explain their significance.		(7)

b.	Prove that the coupling co-efficient between two magnetic circuits is k=ML1L2.	(8)

20.	Explain the following terms:														(5x3)

a.	Maximum value		b.	Average value		c.	Root mean square value

d.	Form factor    		e.	Peak factor

(OR)

21. 	With a neat sketch describe the operation of a thermal power station.

22. 	Explain the construction and operating principle of a DC Generator in detail.

(OR)

23.	a.	Explain the principle of operation and working of a three phase induction motor.		(12)

b.	List out the applications of single phase induction motors.	 						(3)

24.	Describe the working of:

a.	Induction type Energy meter		b. 	Moving coil instrument					(8+7)

(OR)

25.	a.	Describe the operation of staircase wiring with a neat diagram.						(8)

b.	What are the different methods of earthing?  Explain them briefly.					(7)