Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC ELECTRICAL AND ELECTRONICS ENGINEERING														      								       Time: 3 hours

Subject Code: 12EE102								       		      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the condition for Ohm’s law.

2.	Give an example for active and passive elements.

3.	Write down the condition for maximum efficiency in case of a DC generator.

4.	Give the expression for form factor.

5.	Write down the expression and unit of admittance.

6.	List any two losses of DC generators.

7.	State the expression and truth table of 7486 gate.

8.	Define IC.

9.	List the uses of Megger.

10.	What are the features of moving coil instruments?

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the V-I relationship of circuits elements.

12.	Define Power and Power factor.

13.	List any four parts of a DC Machine and write the two types of armature windings.

14.	Define intrinsic and extrinsic semi conductors.

15.	Write any four methods by which deflecting torque can be obtained and mention the two methods of obtaining controlling torque.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the current in each branch in the network shown in the figure belwo.

(OR)

17.	A bridge circuit ABCD has RAB = 2Ω, RBC=3Ω, RCD=40Ω, RDA=1Ω and RBD=5Ω. A Battery of 5V with negligible internal resistance is connected between the terminals A and C. Determine the current in each branch.

18.	Derive the relation between line and phase values of voltage and current in Y connection and in ∆ connection.

(OR)

19.	a.	Derive the expression for RMS value, average value, form factor and peak factor of the 	sinusoidal alternating quantity.	       						       						(8)

[P.T.O]

b.	Find the average value and effective value of a saw tooth wave form shown in the figure 			below.																		(7)

20.	Explain in detail about the construction of DC machine with a neat diagram.

(OR)

21.	Discuss about any five types of single phase induction motor.

22.	Explain V-I characteristics of PN Junction diode.

(OR)

23.	Explain the following:

a.	Zener Diode.								      									(5)

b.	FET. 									      									(5)

c.	SCR.									      									(5)

24.	Describe the Hall Effect and also write notes on Piezo-Electric transducers.

(OR)

25.	Elaborate on moving coil and moving iron instruments.