Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASICS OF MEDICAL ELECTRONICS					      Time: 3 hours

Subject Code: 12EE251			       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	When is a cell said to be depolarized?

2.	What does an imbalance between myocardial oxygen supply and demand result in?

3.	Discriminate a bio-electrode from a conventional electrode.

4.	Which instrument is used to record motor nerve conduction velocity?

5.	What is the main difference between a sensor and a transducer?

6.	Give a medical application of a photo emissive cell.

7.	Where do you deploy a Radio PiLL?

8.	Specify the systolic blood pressure rate.

9.	Which of the two shocks 		a.	current 		b.	voltage 	is fatal?

10.	During Laser surgery, mention the precautionary steps taken to avoid electrical accidents.

PART – B  (5 x 3 = 15 MARKS)

11.	What does the conduction of the breath sounds to the stethoscope denote?

12. 	Bring out the reason for using a glass electrode in a pH electrode.

13.	Distinguish a thermocouple from a thermistor.

14.	Enlist the advantages of a bedside monitor.

15. 	What are the sources of leakage current in a clinical environment?

PART – C  (5 x 15 = 75 MARKS)

16. 	With neat sketches investigate how a cell potential is originated and transmitted. What happens when the path is cut?

(OR)

17. 	Characterize the functioning of the central nervous system.

18.	With neat block diagrams explain how ECG is recorded.

(OR)

19. 	Analyze the characteristics of the various types of electrodes used for recording EMG and EEG respectively.

20. 	Classify the various transducers used in clinical applications.

(OR)

21. 	Discuss the working of an LVDT pressure transducer.

22. 	Explain any one indirect method of measuring blood pressure.

(OR)

23. 	Evaluate the deployment of microprocessors in patient monitoring systems.

24. 	Summarize the various measures undertaken to make the clinical environment an electrical accident free zone.

(OR)

25. 	Generalize the safety standards followed internationally for enhancing the safety aspects of medical instruments.