# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: POWER CONVERTER ANALYSIS – I						 Time: 3 hours

# Subject Code: 12EE302							             			 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Explain in detail the operation of a single phase fully controlled thyristor bridge 	converter feeding RL load. Draw the necessary waveform and derive the expression for 	the average and rms output voltage assuming continuous current operation.		     (12)

b.	A single phase fully controlled bridge converter feeds an RL load having a resistance of 10 Ω and an inductance of a very high value causing perfect smoothing. The converter is fed from a 230V, 50Hz, 1 phase supply. For a firing angle delay of α=300, determine

i.	Average load voltage		ii.	Average load current		iii.	 Input power factor																			(8)

(OR)

2.	a.	Explain the effect of source inductance in the performance of single-phase full 	converter. Derive expression for its output voltage in terms of firing and overlap angle.																			      (10)

b.	With relevant circuit diagram and waveforms, explain in detail the operation of a single phase half controlled thyristor bridge converter feeding RLE load for discontinuous current conduction mode. 													     (10)

3.	a.	With a relevant circuit diagram and waveforms, explain the operation of three phase 	semi converter with resistive load for continuous conduction mode. Also derive the 	expression for average and RMS values of output voltage.						      (12)

b.	A three phase fully controlled converter is operated from a three phase star connected 		208 V, 60 Hz supply and the load resistance is R = 10 Ω. If it is required to obtain an 		average output voltage of 50 % of the maximum possible output voltage, calculate

i.	delay angle  		ii.	rms and average output current 						(8)

(OR)

4.	Draw the circuit diagram of three phase dual converter and explain its principle of operation. Also draw the relevant waveforms and obtain the expression for circulating current.

5.	a.	Draw and explain the circuit diagram of Buck converter with continuous load current and 	also derive the expression for average output voltage, input current, peak-peak ripple 	current of inductor and peak-peak ripple voltage of C.							     (12)

b.	A step down DC chopper is operating at a frequency of 2 kHz from a 96 V DC source to 	supply a load resistance of 8 Ω. The load time constant is 6ms. If the average load 	voltage is 57.6 V, find:

i.	Ton period of Chopper		ii.	RMS value of load Voltage 	iii.	Average Load 		Current		iv.	Magnitude of Ripple Current. 								(8)

(OR)

6.	a.	Explain the operation of the step up chopper with associated output voltage and current 	waveforms for continuous current mode with RL load.  Also derive the expression for 	the peak to peak ripple current. 											     (10)

b.	A buck-boost regulator has an input voltage of Vs = 12V.  The duty cycle k = 0.25 and 	the switching frequency is 25 kHz.  The inductance L = 150µH and filter capacitance     	C = 220 µF.  The average load current Ia = 1.25A.  Calculate:	i.	the average output 	voltage Va	ii.	 The peak to peak output ripple voltage Vc  	iii.	the peak to peak 	ripple current of inductor I	iv.	the peak current of the transistor, Ip.			      (10)

[P.T.O]

7.	a.	Explain about the principles of AC voltage controllers with PWM control.	   		(8)

b.	The single phase full wave controller supplies on RL load.  The input rms voltage is     		Vs =120V, 60 Hz.  The load is such that L = 6.5 mH and R = 2.5.  The delay angles of 		thyristors are equal.  α1 = α2 = π/2.   Determine:

i.	the conduction angle of thyristor T1		ii.	the rms output voltage Vo

iii.	the rms thyristor current IR				iv.	the rms output current Io

v.	the average current of a thyristor IA,		vi.	 the input power factor.     		     (12)

(OR)

8.	With relevant circuit diagram and waveforms, explain the operation of a three phase full wave controller with resistive load.  Derive the expression for the rms output phase voltage in the range of 0o to 60o delay angle.

9.	Compulsory:

a.	Describe the basic principle of a single phase to single phase cyclo-converter for both continuous and discontinuous conductions for a bridge type cyclo-converter.	      (10)

b.	The input voltage to the cyclo converter is 120V rms. 60 Hz. The load resistance is 5 Ω and the load inductance is L=40mH. The frequency of the output is 20Hz. If the converters are operated as semi-converter such that 0≤α≤π,  and the delay  angle is αp=2π/3, determine:

i.	the rms value of output  voltage Vo		ii.	the rms current of each thyristor IR

iii.	the input power factor.										        		        (10)