# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: SOLID STATE AC DRIVES						            	 Time: 3 hours

# Subject Code: 12EE305				         		        		 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	A three phase 11.2 kW, 1750 rpm, 460V, 60 Hz, 4 pole, star connected induction motor 	has the following parameters. Rs=0.66Ω, Rr=0.38Ω, Xs=1.14Ω, Xr=1.71Ω and 	Xm=33.2Ω. The motor is controlled by varying both the voltage and frequency. The 	volts/hertz ratio which corresponds to the rated voltage and rated frequency, is 	maintained constant.

i.	Calculate the maximum torque Tm and corresponding speed ωm for 60 Hz and 30 		Hz.

ii.	Calculate the maximum torque Tm and corresponding speed ωm for 60 Hz and 30 Hz 		if Rs is negligible.										 			(12)

b.	Obtain the expression for the torque developed in induction motor from the equivalent 	circuit referred to stator.													(8)

(OR)

2.	Explain different methods employed for braking of an induction motor with relevant diagrams and waveforms.

3.	A three phase 11.2 kW, 1750 rpm, 460V, 60 Hz, 4 pole, star connected induction motor has the following parameters. Rs=0.66Ω, Rr=0.38Ω, Xs=1.14Ω, Xr=1.71Ω and Xm=33.2Ω. The no load loss is negligible. The motor is controlled by a current source inverter and the input current is maintained constant at 20A. If the frequency is 40 Hz and developed torque is 55 N-m, determine:		a.	the slip for maximum torque sm and maximum torque Tm

b.	the slip, s			c.	the rotor speed ωm	d.	the terminal voltage per phase, Va

e.	power factor PFm.

(OR)

4.	Explain the operation of induction motor from a variable frequency current source with relevant diagrams. Also obtain the expressions for the rotor current, developed torque, starting torque and maximum torque.

5.	A three phase 460V, 60 Hz, 6 pole, star connected wound rotor induction motor is controlled by static Kramer drive. The turns ratio of the converter AC voltage to supply voltage is nc=Na/Nb=0.40. The load torque is 750 N-m at 1175 rpm. If the motor is required to operate at a speed of 1050 rpm, calculate	a.	the inductor current Id		b.	the DC voltage Vd

c.	the delay angle of the converter, α	 	d.	the efficiency	e.	the input power factor of the drive, PFs. The losses in the diode rectifier, converter, transformer and inductor Ld are negligible.

(OR)

6.	a.	The line Power factor of a static Kramer drive is poor. Why? Can you suggest the 	method to improve line power factor of a static Kramer Drive.					(10)

b.	Explain static rotor resistance control in induction motor with relevant diagrams and 	waveforms.   															(10)

7.	Compulsory:

How do you control the speed and torque of Induction Motor using indirect vector controlled scheme. Also explain the principle of flux weakening operation.

[P.T.O]

8. 	a.	A thee phase 460V, 60Hz, 6 pole, star connected cylindrical rotor synchronous motor has 	a synchronous reactance of Xs=2.5Ω and the armature resistance is negligible. The load 	torque, which is proportional to the speed squared, is TL=398 Nm at 1200 rpm. The 	power factor is maintained at unity by field control and the voltage to frequency ratio is 	kept constant at the rated value.  If the inverter frequency is 36 Hz and the motor speed is 	720 rpm, calculate:

i.	the input voltage Va	ii.	the armature current Ia		iii.	the excitation voltage Vf 	iv.	the torque angle ∂		v.	the pull out torque Tp.							(10)

b.	Explain margin angle control in synchronous motor with relevant diagrams and 	waveforms.																(10)

(OR)

9. 	a.	A 6 MW, 3 phase, 11kV, star connected 6 pole, 50 Hz, 0.9 leading p.f. synchronous 	motor has a synchronous reactance of 9 ohm and negligible resistance. Rated field 	current is 50 A. The machine is controlled by a variable frequency control at a constant 	V/f 	ratio up to the base speed and at constant speed above the base speed. Determine the 	torque and field current for rated armature current, 7500 rpm and 0.8 leading power 	factor.																	(10)

b.	Explain the self controlled mode of operation of synchronous motor drive employing a 	cycloconverter with relevant circuit and waveforms.							(10)