# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: POWER ELECTRONIC CIRCUITS						 Time: 3 hours

# Subject Code: 12EE314					               			 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Briefly discuss the V-I characteristics of SCR.								(10)

b.	Explain the turn on and turn off characteristics of MOSFET.					(10)

2.	a.	Describe the operation of a three phase full converter. Also derive the expression for its 	average output voltage.													(10)

b.	A three-phase full-wave converter is operated from a three-phase Y –connected 208-V, 60-Hz supply and the load resistance is R=10Ω. If it is required to obtain an average output voltage of 50% of the maximum possible output voltage, calculate:

a.	the delay angle α		b.	the rms and average output currents		c.	the average and rms thyristor currents.													(10)

(OR)

3.	a.	Explain the operation of a single phase dual converter. Also derive the expression for its 	average output voltage.													(10)

b.	A single-phase dual converter is operated from a 120-V, 60-Hz supply and the load 	resistance is R=10Ω. The circulating inductance is Lr=40mH; delay angles are α1= 60° 	and α2= 120°. Calculate the peak circulating current and the peak current of converter1.

(10)

4.	a.	Describe the working principle of a boost converter with necessary circuit and

waveforms.																(10)

b.	A boost regulator has an input voltage of Vs=5V.The average output voltage Va=15V and the average load current Ia=0.5A.The switching frequency is 25 kHz. If L=150 µH and C=220 µF. Determine:	 a.	the duty cycle k	b.	the ripple current of inductor ∆I.																		(10)

(OR)

5.	a.	Discuss the function of a buck-boost converter with a neat circuit diagram and 				waveforms.																(10)

b.	A buck-boost regulator has an input voltage of Vs=12V.The duty cycle k=0.25 and the switching frequency is 25 kHz. The inductance, L=150 µH and filter capacitance,    C=220 µF. The average load current is Ia=1.25A. Determine:

a.	the output voltage , Va		b.	the peak-to-peak output voltage ripple, ∆Vc.	(10)

6.	a.	Explain the principle of operation of a three phase voltage source inverter with 180° 	conduction mode with necessary circuit diagram and wave form.				(10)

b.	Discuss the voltage control techniques of single-phase inverters with diagrams.	(10)

(OR)

7.	a.	Draw the power circuit and waveforms of quasi resonant converter and explain the 	operation. 																(10)

b.	Explain the multi resonant converter with necessary circuit and waveforms. 		(10)

8.	a.	Illustrate how the output voltage is varied in both cycles of single phase 50 Hz AC input 	using AC voltage controller. Draw the necessary circuit and waveforms.		(10)

[P.T.O]

b.	A single phase full-wave AC voltage controller has a resistive load of R=10Ω  and the 	input voltage is Vs=120V (rms),60Hz.The delay of thyristor T1 and T2 are  	α1=α2=α=𝝅/2.Determine:

a.	the rms output voltage Vo		b.	the input PF.							(10)

(OR)

9.	a.	Describe the working principle of three phase cyclo-converters.				(10)

b.	What are the effects of source and load inductances in AC voltage regulators?	(10)