# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: DESIGN OF TURBINES FOR RENEWABLE ENERGY SYSTEMS																				 Time: 3 hours

# Subject Code: 12EE346				         		        		 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- Consider a wind turbine with 5 m diameter rotor. Speed of the rotor at 10 m/s wind velocity is 130 r/min and its power coefficient at this point is 0.35. Calculate the tip speed ratio and torque coefficient of the turbine. What will be the torque available at the rotor shaft? Assume the density of air to be 1.24 kg/m3.								(10)

- For Rann of Kutch the basin area of a tidal project is 0.72 sq. km, with a difference of   6 m between the high and low water levels. The average available head is 5 m and the system generates electric power for 4 hours in each cycle. Assuming the overall efficiency as 80%, calculate the power in kW at any point of time and the yearly power output. Density of sea water is 1025 kg/m3.									(10)

2.	a.	Calculate the loss resistance of the evacuated collector which has outer diameter             		D = 2 cm and inner diameter = 1 cm. Total isolation on surface of collector is 750 W/m2. 		Assume ambient temperature to be 20◦C and wind velocity = 5 m/sec. Mean plate 			temperature may be assumed equal to 100◦C. Assume εp = 0.1 and εg = 1.		(10)

b.	Draw a layout of solar pond power plant and explain. What are its main applications?

(10)

(OR)

3.	a.	Explain the different types of losses in solar cells.  							(10)

b.	A solar cell short-circuit current is 35 mA/cm2, open-circuit voltage is 0.7 V, fill factor is 65 % and cell area is 100 cm2 then find the short circuit current (ISC) and maximum power point (Pm) for different illumination level for a cell of efficiency 16%. The incident sunlight varies as 1000, 600 and 200 W/m2. Comment about output current and power of the cell.															(10)

4.	a.	With the help of a neat sketch, discuss the different types of rotors used in wind turbines 	and also draw the variation of Cp with λ for different rotors. 					(10)

b.	Design the rotor radius for a multiblade wind turbine that operates in a wind speed of   36 kmph to pump water at a rate of 6 m3/h with a lift of 6 m. Also, calculate the angular velocity of the rotor. Given: water density = 1000 kg/m3, g = 9.8 m/s, water pump efficiency = 50%, efficiency of rotor to pump = 80%, Cp = 0.3, λ = 1.0, and air density is 1.2 kg/m3.																(10)

(OR)

5.	a.	Prove that in case of horizontal axis wind turbine maximum power can be obtained 	when: Exit velocity = 13wind velocity andPmax=827ρAV3.					(10)

b.	Wind at 1 standard atmospheric pressure and 15°C temperature has a speed of 10 m/s.    A 10 m diameter wind turbine is operating at 5 rpm with maximum efficiency of 40%. Calculate:

i.	the total power density in wind stream,

ii.	the maximum power density,

iii.	the actual power density,

[P.T.O]

iv.	the power output of the turbine, and

v.	the axial thrust on the turbine structure.									(10)

6.	Draw and explain the different types of hydro turbines. List out their advantages and disadvantages.

(OR)

7.	a.	With the help of a neat schematic diagram, discuss in detail the utilities of the basic 	components used in a typical small hydroelectric power plant. 					(10)

b.	Water flows through a turbine at a rate of 10,000 kg/s. Operating water head is 50 m. 	The system overall conversion efficiency is 0.85. The turbine mechanical efficiency is 	0.99. The generator efficiency is 0.96. The discharge velocity of water is 2.5 m/s. 	Determine :

i.	The turbine-generator power output.

ii.	The turbine internal power.

iii.	The fluid head loss. 													(10)

8.	a.	Lockheed OTEC project is of the closed cycle type using ammonia as the working fluid. 	The overall temperature difference is 18.46 K. The warm water flow rate is 341.6 m3/s. It 	enters the heat exchanger at 26.53◦C and exists at 25.17◦C, having been cooled by       	1.36 K. The ammonia at the turbine inlet is at 23.39◦C while at the outlet is 11.11◦C. The 	pressure of the ammonia at the inlet to the turbine is 0.96 MPa and at the outlet is        	0.64 MPa. Electricity is produced at 90.2% of Carnot efficiency. Calculate:

i.	Thermal power delivered by warm water to the OTEC.

ii.	Carnot efficiency

iii.	Electrical power generated

iv.	Electrical power at output bus, if internal power used in the OTEC is 19.55 MWe.

v.	The overall efficiency of OTEC system

vi.	Energy rejected to cold water.											(10)

b.	A single-basin single-effect tidal power plant has a basin of area 40 x 106 m2 and range 		of 12.5 m. The turbine stops operation when the head on it falls below 2.5 m. Calculate 		the average energy generated in one tidal cycle in KWh, if the turbo generator efficiency 		is 0.75%.																(10)

(OR)

9.	a.	A binary fluid (water/isobutane) geothermal power plant uses geothermal water at 148◦C 	and leaves at 70◦C. The turbine inlet condition is 120◦C, dry saturated. Pump internal 	work is -9.55 kJ/kg. Isobutane flow rate in the turbine is 100 kg/s. Pressure of isobutane 	in the condenser is 0.4 MPa. Turbine internal efficiency is 0.9. Mechanical efficiency of 	turbine is 0.98. Generator efficiency is 0.95. Pump motor efficiency is 0.94. Calculate:   	i. 	The geothermal water flow rate and 	ii.	 Electric power output. 		(15)

b.	Draw a layout of flashed steam geothermal power plant. What are its advantages and 			disadvantages?															(5)