Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC ELECTRICAL ENGINEERING					       Time: 3 hours

Subject Code: 12EE101							      		   		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Ohm’s law.

2.	What will be the resistance of a 60 W, 250 V, incandescent lamp?

3.	The opposition offered to the flow of flux is called ___________.

4.	Give the relation between self inductance, mutual inductance and coupling co-efficient.

5.	Define form factor.

6.	Write the equation of sinusoidal alternating voltage with maximum value Vm and frequency ω.

7.	The principle of operation of a transformer is ___________.

8.	A Single phase induction motor is non self-starting – Yes or No.

9.	Mention any two types of wiring.

10.	Name the instrument used for measuring insulation resistance.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the branches in the figure below to single voltage source in series with a single 	resistor.

1A

2A

1Ω

2Ω

12.	Determine the voltage VAB in figure.

A

15 V

22KΩ

100KΩ

B

13.	State Faraday’s laws of electromagnetic induction.

14.	Compare OHL and UGC for transmission of electrical power.

15.	Draw and explain the wiring layout for stair case wiring.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Two 50 Ω resistors are connected in series. When a resistor R is connected across one of 	them, the total circuit resistance is 60 Ω. Calculate the value of R.

[P.T.O.]

b.	Find the voltage across A and B in the circuit shown in the figure below.

10Ω

4Ω

6 V

6Ω

12V

4Ω

12V

A

B

+

+

+

-

-

-

(OR)

17.	a.	Find the equivalent resistance between the terminals A and B of the figure shown below.

1Ω

1Ω

2Ω

A

B

2Ω

1Ω

b.	Classify the circuit elements.

18.	a.	Write notes on magnetic materials.

b.	Calculate the magneto motive force required to produce a flux of 0.02 Wb across an air 	gap 3 mm long, having an effective area of 300 cm2.

(OR)

19.	a.	Define self inductance and mutual inductance.

b.	Derive an expression for equivalent inductance of two coils connected in series.

20.	Obtain the average value, rms value, form factor and peak factor for the waveform shown in 	the figure below.

v

Vm

π

0

(OR)

21.	With suitable diagrams, explain a nuclear power generating station.

22.	Discuss the principle of operation of a DC motor. Mention few applications of DC motors.

(OR)

23.	Describe the construction and principle of operation of a three phase induction motor.

24.	a.	Write a note on the different types of torques acting on measuring instruments.

b.	Explain the operation of a moving coil instrument.

(OR)

25.	a.	Draw and explain about florescent tube wiring.

b.	Discuss the need for earthing.